TECHNICAL AND ECONOMIC POTENTIAL OF DEMAND RESPONSE

A Design Project

Presented to the Faculty of the Graduate School

of Cornell University

In Partial Fulfillment of the Requirements for the Degree of

Master of Engineering

by

Grace Mitchell

November 2017

© 2017 Grace Mitchell

ABSTRACT

DNV GL, an international company that offers engineering consulting services, was approached by three utility companies in New York: New York State Electric & Gas, Rochester Gas and Electric Corporation, and National Grid plc, to complete an analysis titled “New York Technical Potential Evaluation Study.” DNV GL approached the Anderson lab group to complete sections of the study, which spans the years of 2017 to 2027. The analysis includes the technical and economic potential of energy conservation measures that have been considered promising for New York: energy efficiency technologies, renewable energy, and Demand Response programs. This Design Project includes only the results of the analysis responsible for the technical and economic potential of the DR programs. For each utility, commercial programs are economically viable for every year. Residential programs had varying results across the three utilities, and should only be implemented going forward with consideration of recommended improvements.

INTRODUCTION

DNV GL is an international accredited registrar and classification society. In addition to completing audits and issuing certifications, DNV GL offers engineering consulting services. For energy consulting in particular, the company offers analysis, strategies, and implementation services which help entities become more energy efficient or sustainable. The company was approached by three utilities in New York: New York State Electric & Gas (NYSEG), Rochester Gas and Electric Corporation (RG&E), and National Grid plc, to complete an analysis titled “New York Technical Potential Evaluation Study.” DNV GL then approached the Anderson lab group, as well as other faculty at Cornell University, to complete sections of the study.

The study includes an analysis of both the technical and economic potential of energy conservation measures that have been considered promising for the state of New York: energy efficiency technologies, renewable energy systems, and Demand Response (DR) programs. The analysis spans each year from the current year, 2017, to 2027. The purpose of this study is to help the utility companies meet New York State policy requirements, as defined by Governor Cuomo’s Reforming the Energy Vision (REV) Strategy. This Design Project includes only the results of the analysis responsible for the technical and economic potential of the DR programs, which are a way for utilities to mitigate energy demand in an effort to maintain grid reliability. Utilities essentially ask their customers enrolled in DR programs to shed load during periods of high demand.

As defined by DNV GL in their Request for Proposal, technical potential is the “complete penetration of all measure applications deemed technically feasible from an engineering perspective,” and economic potential is the “technical potential of those energy conservation measures that are cost-effective when compared to supply-side alternatives.”

The technical potential of DR programs is based upon the programs currently available at each utility. The analysis assumes that current DR programs are feasible for each year until 2027, that all utility customers are eligible to enroll, but not all of them will. The framework of the analysis includes the evaluation of current programs, which includes estimates of current enrollment and current electricity relief in kW, and an evaluation of program potential, which includes estimates of potential enrollment and potential electricity relief in kW.

The economic potential considers the tradeoffs between the cost of incentives paid by the utility, and the Avoided Generation Capacity Costs (AGCC) due to 1 kW reduction in peak load. The analysis assumes all DR events take place during times of peak load, and that each event lasts for one hour. The analysis also considers the implementation of energy efficiency technology, as an overall reduction in potential generation. Whichever program implementation provides positive economic gain is considered economically viable.

DR TECHNICAL POTENTIAL: EVALUATION OF CURRENT PROGRAMS

Current Program Description

NYSEG and RG&E

| Programs | Bring Your Own Thermostat (BYOT) and Direct Install Thermostat (DIT) |
| --- | --- |
| Description | - This voluntary demand response program is designed to reward customers for reducing electricity use during periods of high demand for electricity. Participants agree to brief, limited events which adjust their thermostats during peak electric demand periods from May 1 through September 30 on non-holiday weekdays. You can anticipate at least one event, and a maximum of up to 15 adjustments per summer. Thermostats will be automatically adjusted during events. |
| Requirements | - Participants must be an electric customer, but not an hourly pricing customer, with an installed, eligible internet-connected thermostat used to control their central air conditioning. |
| Incentives | - Participants will receive an $85 Amazon e-gift card after being accepted into the program and a $5 bill credit for each event they fully participate in. |

| Program | RoomAC |
| --- | --- |
| Description | With electricity system consumption peaking in the summer, the smartAC device allows the utility to manage room air conditioning loads, thereby reducing the summertime peak, the stress on the electricity system and the cost everyone pays for electricity. Typically, the utility comes close to peak load about 4 or 5 times a year for about 3 to 4 hours. When this happens, the utility would notify the participant and, if necessary, could manually raise their air conditioner temperature setting by a few degrees to reduce the overall peak load. Participants are able to manually override this higher setting. |
| Requirements | Participants must install the smartAC kit. |
| Incentives | Participants receive a smartAC kit, which includes a remote control, instructions on how to set it up, and a $25 gift card. |

| Program | Commercial System Relief Program (CSRP) |
| --- | --- |
| Description | Participants are paid to save energy, as they play a significant role in the operation of the electricity grid by reducing or shifting their electricity use during peak periods. The Commercial System Relief Program, and other similar demand response programs, can lower the cost of electricity in wholesale markets and, in turn, lead to lower retail rates. The CSRP provides notification of up to 21 hours to enrolled customers to reduce their electricity use to help maintain system reliability in their communities. |
| Requirements | Commercial customers can either apply through an aggregator, or through the utility as a direct participant if they are able to reduce their electricity use by 50 kW or more. There is a Voluntary Participation Option or a Reservation Payment Option. Under the Voluntary Participation Option, the utility will make a payment to a direct participant or aggregator who provides load relief during a designated load relief period. To participate in the Reservation Payment Option, the direct participant or the aggregator must commit to load relief for planned and unplanned events upon our request. |
| Incentives | For the Voluntary Participation Option, the performance amount paid per event is equal to the applicable performance payment rate per kilowatt-hour (kWh) multiplied by the average kilowatt (kW) of load relief provided during the event multiplied by the number of event hours. The current performance payment rate is $0.15 per kWh. For the Reservation Payment Option, the monthly payment is calculated by multiplying the applicable reservation payment rate per kW by the kW of contracted load relief multiplied by performance factor for the month. There is a reservation payment rate for months where we have requested four or fewer planned load relief events. There is also a reservation payment rate for months where we have requested five or more planned load relief events. The current reservation payment rate for four or less planned events is $3.25 per kW per month. The current reservation payment rate for five or more planned events is $3.50 per kW per month. |

| Program | Distribution Load Relief Program (DLRP) |
| --- | --- |
| Description | Participants are paid to save energy, as they play a significant role in the operation of the electricity grid by reducing or shifting their electricity use during peak periods. The Distribution Load Relief Program, and other similar demand response programs, can lower the cost of electricity in wholesale markets and, in turn, lead to lower retail rates. The DLRP provides two hours or less notification to enrolled customers to reduce their electricity use to help maintain system reliability in their communities. |
| Requirements | Commercial customers can either apply through an aggregator, or through the utility as a direct participant if they are able to reduce their electricity use by 50 kW or more. There is a Voluntary Participation Option or a Reservation Payment Option. Under the Voluntary Participation Option, the utility will make a payment to a direct participant or aggregator who provides load relief during a designated load relief period. To participate in the Reservation Payment Option, the direct participant or the aggregator must commit to load relief for no less than four consecutive hours upon our request. |
| Incentives | For the Voluntary Participation Option, the performance amount paid per event is equal to the applicable performance payment rate per kilowatt-hour (kWh) multiplied by the average kilowatt (kW) of load relief provided during the event multiplied by the number of event hours. The current performance payment rate is $0.15 per kWh. For the Reservation Payment Option, the monthly payment is calculated by multiplying the applicable reservation payment rate per kW by the kW of contracted load relief multiplied by performance factor for the month. For a load reduction requested by us, the direct participant or the aggregator will receive a performance payment per kWh for the first four hours of load relief provided during the load relief period. There is also a performance payment rate for requested load relief period hours over four hours. The current reservation payment rate for four or less planned events is $3.25 per kW per month. The current reservation payment rate for five or more planned events is $3.50 per kW per month. The current performance payment rate for the first four hours of the load relief period is $0.15 per kWh. The current performance payment rate for the load relief period hours over four hours is $0.30 per kWh. |

National Grid

| Program | Emergency Demand |
| --- | --- |
| Description | This program is used when the NYISO declares a system emergency. Companies enrolled in this program will receive a financial incentive if they can curtail at least 100 kW of electricity one hour after notification. Incentive payments will only be made to program participants if power use is actually curtailed. |
| Requirements | - Commercial participants must be able to curtail at least 100 kW, as well as meet rate class, interval metering and on-site generation requirements. |
| Incentives | The NYISO will pay National Grid the greater of $500 MWH or the real time Location Based Marginal Price (LBMP). National Grid will pay 90 percent of the NYISO payment for your company's actual kWh reduction. |

| Program | Day Ahead Demand |
| --- | --- |
| Description | This program is used when customer bids to reduce load are accepted by the NYISO, usually when day ahead electricity prices are forecast to be high. This provides economic incentives to reduce electricity demand when prices are high. Companies will supply load reduction bids that we will submit to the NYISO or aggregate with other customer bids and submit to the NYISO. Bid submissions will include the level of reduction available (inclusive of losses), the time period, and customer specification for the Day-Ahead Location Based Marginal Price above which they will reduce their load. Bids may be revised once per week, and companies can elect not to participate during any given week by submitting a 'no bid' request. If the consumer bid is accepted by the NYISO, National Grid will notify you by 5 p.m. prior to the day of curtailment. There are financial penalties associated with non-performance. |
| Requirements | Commercial participants must be able to curtail at least 100 kW, as well as meet rate class and interval metering requirements. |
| Incentives | The NYISO will pay National Grid the day ahead LBMP times the customers’ bid reduction. National Grid will pay participating customers 90 percent of the NYISO payment for their bid reduction. Payments will only be made for load reductions that are not provided from the use of on-site generation. |

Estimates of Current Enrollment and Relief

NYSEG

|  | Program |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BYOT/DIT | RoomAC | CSRP | DLRP | Total |
| Year | 2017 | 2017 | 2017, 2016, 2015 | N/A | N/A |
| Customer Type | Residential | Residential | Commercial | Commercial | N/A |
| Device Description | Thermostats | smartAC Control | Electric Meters | Electric Meters | N/A |
| Number of Events | 1 | 1 | 0 in 2017, 2 in 2016, Test in 2015 | N/A | 2 in 2017, 2 in 2016, Test in 2015 |
| Number of Enrolled and Equipped Devices | 2009 | 577 | 51 in 2017, 61 in 2016, 12 in 2015 | N/A | 2637 in 2017, 61 in 2016, 12 in 2015 |
| Number of Devices/Participants Participating in Event(s) | 1,761 Devices | 221 Devices | 51 Devices and 2 Participants in 2017, 61 Devices and 7 Participants in 2016, 12 Devices and 8 Participants in 2015 | N/A | 2,033 Devices and 1,984 Participants in 2017, 61 Devices and 7 Participants in 2016, 12 Devices and 8 Participants in 2015 |
| Average kW Relief per Event | 1,190 | 0.084 | 0 in 2017, 14,928 in 2016, 6,021 in 2015 | N/A | 1,190 in 2017, 14,928 in 2016, 6,021 in 2015 |
| Estimated kW Relief per Device per Event | 0.676 | 0.0003 | 373 | N/A | N/A |

RG&E

|  | Program |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BYOT/DIT | RoomAC | CSRP | DLRP | Total |
| Year | 2017 | 2017 | 2017, 2016, 2015 | N/A | N/A |
| Customer Type | Residential | Residential | Commercial | Commercial | N/A |
| Device Description | Thermostats | smartAC Control | Electric Meters | Electric Meters | N/A |
| Number of Events | 1 | 1 | 0 in 2017, 13 in 2016, Test in 2015 | N/A | 2 in 2017, 13 in 2016, Test in 2015 |
| Number of Enrolled and Equipped Devices | 1,752 | 336 | 17 in 2017, 35 in 2016, 4 in 2015 | N/A | 2,105 in 2017, 35 in 2016, 4 in 2015 |
| Number of Devices/Participants Participating in Event(s) | 1,690 Devices | 167 Devices | 17 Devices and 3 Participants in 2017, 35 Devices and 4 Participants in 2016, 4 Devices and 1 Participant in 2015 | N/A | 1,874 Devices and 1,860 Participants in 2017, 35 Devices and 4 Participants in 2016, 4 Devices and 1 Participant in 2015 |
| Average kW Relief per Event | 1,497 | 0.084 | 0 in 2017, 9,938 in 2016, 694 in 2015 | N/A | 1,497 in 2017, 9,938 in 2016, 694 in 2015 |
| Estimated kW Relief per Device per Event | 0.886 | 0.0005 | 229 | N/A | N/A |

As no data for National Grid’s DR programs is available, the programs available through NYSEG and RG&E will be assumed possible for National Grid. For National Grid, the technical results will be extrapolated, using estimations of their customer count and an average of the results obtained through the technical analysis of NYSEG and RG&E.

National Grid

|  | Program |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BYOT/DIT | RoomAC | CSRP | DLRP | Total |
| Average kW Relief per Event | 1,344 | 0.084 | 4,114 | N/A | 5,458 |
| Estimated kW Relief per Device per Event | 0.781 | 0.0004 | 301 | N/A | N/A |

DR TECHNICAL POTENTIAL: EVALUATION OF PROGRAM POTENTIAL

Estimates of Potential Enrollment

For the evaluation of technical potential, it will be assumed that all current programs will remain active until 2027. Potential programs will be discussed at the end of this section.

Estimate of NYSEG Current Customer Count

| Customer Type | Count |
| --- | --- |
| Residential | 686,499 |
| Commercial | 102,476 |
| Industrial | 135 |
| Municipal | 13,397 |

Estimate of RG&E Current Customer Count

| Customer Type | Count |
| --- | --- |
| Residential | 330,920 |
| Commercial | 38,457 |
| Industrial | 723 |
| Municipal | 2,741 |

Estimate of National Grid Current Customer Count

| Customer Type | Count |
| --- | --- |
| Residential | 2,079,352 |
| Commercial | 30,263 |
| Industrial | 25,739 |
| Municipal | 1,114 |

The technical analysis will assume that all customers are eligible to enroll in their respective DR programs, but not all of them will. Residential customers will be assumed eligible for the designated residential programs (BYOT, DIT, and RoomAC). Commercial, industrial, and municipal customers will be assumed eligible for all commercial programs (CSRP and DLRP). For example, an estimate of 5 percent enrollment for residential programs means that 5 percent of current residential customers have hypothetically enrolled in DR residential programs. This represents the upper bound, or maximum, number of enrolled participants for any current program based on the current number of customers for each utility. 5, 10, 15, 25, and 50 percent enrollment have been chosen to represent a realistic range of technical potential. For commercial programs, 1, 2, 5, 10, 15 percent enrollment levels will be considered.

NYSEG Potential Enrollment

|  | Programs |  |
| --- | --- | --- |
|  | Residential | Commercial |
| 1% Enrollment | N/A | 1,160 |
| 2% Enrollment | N/A | 2,321 |
| 5% Enrollment | 34,324 | 5,800 |
| 10% Enrollment | 68,649 | 11,600 |
| 15% Enrollment | 102,974 | 17,401 |
| 25% Enrollment | 171,624 | N/A |
| 50% Enrollment | 343,249 | N/A |

RG&E Potential Enrollment

|  | Programs |  |
| --- | --- | --- |
|  | Residential | Commercial |
| 1% Enrollment | N/A | 420 |
| 2% Enrollment | N/A | 839 |
| 5% Enrollment | 16,546 | 2,096 |
| 10% Enrollment | 33,092 | 4,192 |
| 15% Enrollment | 49,638 | 6,288 |
| 25% Enrollment | 82,730 | N/A |
| 50% Enrollment | 165,460 | N/A |

National Grid Potential Enrollment

|  | Programs |  |
| --- | --- | --- |
|  | Residential | Commercial |
| 1% Enrollment | N/A | 572 |
| 2% Enrollment | N/A | 1,143 |
| 5% Enrollment | 103,968 | 2,856 |
| 10% Enrollment | 207,935 | 5,712 |
| 15% Enrollment | 311,903 | 8,568 |
| 25% Enrollment | 519,838 | N/A |
| 50% Enrollment | 1,039,676 | N/A |

Estimates of Potential Relief

As there is no data available for the DLRP, the technical analysis will not attempt to estimate the kW per event of this program. All potential commercial customers will be considered for the CSRP only. As one participant can represent numerous electric meters in the commercial programs, the average number of devices per participant was calculated for each year of data (2015, 2016, and 2017) and averaged to represent the overall average number of devices per participant for the CSRP for each utility. NYSEG has an average of 11 devices per participant for the CSRP. RG&E has an average of 6 devices per participant for the CSRP. NYSEG has an average of 1.24 devices per participant for the BYOT program, and 1.43 devices per participant for the DIT program. RG&E has an average of 1.06 devices per participant for the BYOT program, and 1.01 devices per participant for the DIT program. The RoomAC program includes one device per customer.  To remain conservative in estimates of kW relief per event, it is assumed that there is one device per enrolled participant for all residential programs.

As commercial customers can either apply directly or through an aggregator, it is assumed that 90% of the commercial customers will go through an aggregator, at an average of 100 commercial customers per aggregator. These assumptions are arbitrary, as there is no data available to confirm or deny how many customers apply through an aggregator and on average how many customers each aggregator represents. This is in an effort to represent a realistic estimate of the upper bound of the technical potential.

As it is presumptuous to assume that all residential participants would enroll in all available programs, the residential participants will be allotted appropriately in an attempt to represent a realistic upper bound of the technical potential. For NYSERDA residential programs, 63% of the total residential participants are enrolled in the BYOT/DIT programs, and 37% of total residential participants are enrolled in the RoomAC program. For RG&E residential programs, 77% of the total residential participants are enrolled in the BYOT/DIT programs, and 23% of total residential participants are enrolled in the RoomAC program. For the residential programs of each utility, it will be assumed that each residential customer can only enroll in one program, and that current enrollment trends remain consistent.

NYSEG

| Estimated kW Relief per Event |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Program |  |  |  |
|  | BYOT/DIT | RoomAC | CSRP | Total |
| 1% Enrollment | N/A | N/A | 481,011 | N/A |
| 2% Enrollment | N/A | N/A | 962,022 | N/A |
| 5% Enrollment | 14,505 | 3.86 | 2,405,056 | 2,419,565 |
| 10% Enrollment | 29,011 | 7.72 | 4,810,112 | 4,839,130 |
| 15% Enrollment | 43,516 | 11.58 | 7,215,582 | 7,259,110 |
| 25% Enrollment | 72,528 | 19.30 | N/A | N/A |
| 50% Enrollment | 145,056 | 38.60 | N/A | N/A |

RG&E

| Estimated kW Relief per Event |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Program |  |  |  |
|  | BYOT/DIT | RoomAC | CSRP | Total |
| 1% Enrollment | N/A | N/A | 57,708 | N/A |
| 2% Enrollment | N/A | N/A | 116,790 | N/A |
| 5% Enrollment | 11,241 | 1.93 | 290,870 | 302,113 |
| 10% Enrollment | 22,482 | 3.86 | 581,741 | 604,227 |
| 15% Enrollment | 33,723 | 5.79 | 872,611 | 906,340 |
| 25% Enrollment | 56,205 | 9.65 | N/A | N/A |
| 50% Enrollment | 112,411 | 19.29 | N/A | N/A |

National Grid

| Estimated kW Relief per Event |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Program |  |  |  |
|  | BYOT/DIT | RoomAC | Emergency/ Day Ahead Demand | Total |
| 1% Enrollment | N/A | N/A | 295,735 | N/A |
| 2% Enrollment | N/A | N/A | 590,953 | N/A |
| 5% Enrollment | 56,512 | 12.48 | 1,476,607 | 1,533,132 |
| 10% Enrollment | 113,024 | 24.95 | 2,953,214 | 3,066,263 |
| 15% Enrollment | 169,536 | 37.43 | 4,429,821 | 4,599,394 |
| 25% Enrollment | 282,559 | 62.38 | N/A | N/A |
| 50% Enrollment | 565,118 | 124.76 | N/A | N/A |

Discussion of Potential Programs

Natural Gas Demand Response (DR)

The polar vortex events during the winter of 2014 first introduced the need for natural gas DR programs, as installed pipeline infrastructure did not have the capacity to meet the heating demands of customers.  Some of the affected utility companies have initiated gas DR programs, in the form of trading cheaper gas rates, or interruptible rates, to the customer in exchange for the ability to completely shut off their gas supply when there is too much stress on pipelines. Xcel Energy currently has an interruptible gas program for large commercial and industrial customers, where customers get one hour notice before the DR event and have the option to opt out before the event occurs. SoCalGas’ Advisory Thermostat Program provides another example of a gas DR program, but with a different approach. This program requires the installation of smart thermostats, which allows the utility to control a graduated reduction in gas use during times of significant stress on pipelines.

The answer to the question of whether the potential implementation of gas DR programs for NYSERDA, RG&E, and National Grid is worthwhile or not, depends on many factors: customer willingness to participate, incentives, program requirements, the likelihood of extreme weather events during winter months, and the overall cost of meeting natural gas demand. Depending on these factors, it may or may not be profitable for the utility companies to consider implementing gas DR programs.

DR ECONOMIC POTENTIAL: ECONOMIC VIABILITY BY PROGRAM

Avoided Generation Capacity Costs (AGCC) due to 1 kW reduction in Peak Load

|  | AGCC |  |  |  |
| --- | --- | --- | --- | --- |
| Year | Transmission Level | Distribution Level | Inflation Factor | Total with Inflation |
| 2017 | $67.79 | $67.35 | 1.020 | $137.88 |
| 2018 | $54.92 | $60.27 | 1.043 | $120.10 |
| 2019 | $52.64 | $56.23 | 1.066 | $116.06 |
| 2020 | $43.25 | $49.86 | 1.088 | $101.35 |
| 2021 | $75.83 | $69.14 | 1.111 | $161.05 |
| 2022 | $76.75 | $70.15 | 1.134 | $166.61 |
| 2023 | $79.48 | $73.14 | 1.158 | $176.80 |
| 2024 | $81.95 | $75.62 | 1.183 | $186.36 |
| 2025 | $85.88 | $79.62 | 1.207 | $199.75 |
| 2026 | $90.40 | $84.03 | 1.233 | $215.07 |
| 2027 | $93.83 | $87.85 | 1.258 | $228.56 |

BYOT and DIT Programs

| NYSEG |  |  |
| --- | --- | --- |
| Enrollment | Residential Count | Estimated kW Relief per Event |
| 5% | 34,324 | 14,505 |
| 10% | 68,649 | 29,011 |
| 15% | 102,974 | 43,516 |
| 25% | 171,624 | 72,528 |
| 50% | 343,249 | 145,056 |

| RG&E |  |  |
| --- | --- | --- |
| Enrollment | Residential Count | Estimated kW Relief per Event |
| 5% | 16,546 | 11,241 |
| 10% | 33,092 | 22,482 |
| 15% | 49,638 | 33,723 |
| 25% | 82,730 | 56,205 |
| 50% | 165,460 | 112,411 |

| National Grid |  |  |
| --- | --- | --- |
| Enrollment | Residential Count | Estimated kW Relief per Event |
| 5% | 103,968 | 56,512 |
| 10% | 207,935 | 113,024 |
| 15% | 311,903 | 169,536 |
| 25% | 519,838 | 282,559 |
| 50% | 1,039,676 | 565,118 |

| Event Description | At least one event, and a maximum of up to 15 adjustments per summer. Thermostats will be automatically adjusted during events. |
| --- | --- |
| Requirements | - Participants must be an electric customer, but not an hourly pricing customer, with an installed, eligible internet-connected thermostat used to control their central air conditioning. |
| Incentives | - Participants will receive an $85 Amazon e-gift card after being accepted into the program and a $5 bill credit for each event they fully participate in. |

| Cost |  |
| --- | --- |
| Initial (per participant) | Potential (per participant per event) |
| $85 | $5 |

| NYSEG - for 1 event in 2017 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $137.88 | $1,999,989.68 | $107,286.96 | $1,637,473.34 | $255,229.38 |
| 10% | $137.88 | $4,000,117.25 | $214,577.05 | $3,461,404.82 | $324,135.38 |
| 15% | $137.88 | $6,000,106.93 | $321,867.14 | $5,285,336.30 | $392,903.50 |
| 25% | $137.88 | $10,000,362.07 | $536,447.31 | $8,933,199.25 | $530,715.51 |
| 50% | $137.88 | $20,000,724.14 | $1,072,897.74 | $18,052,856.64 | $874,969.75 |

| NYSEG - for 1 event in 2018 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $120.10 | $1,742,088.50 | $107,286.96 | $1,637,473.34 | $(2,671.80) |
| 10% | $120.10 | $3,484,297.10 | $214,577.05 | $3,461,404.82 | $(191,684.77) |
| 15% | $120.10 | $5,226,385.60 | $321,867.14 | $5,285,336.30 | $(380,817.83) |
| 25% | $120.10 | $8,710,802.80 | $536,447.31 | $8,933,199.25 | $(758,843.76) |
| 50% | $120.10 | $17,421,605.60 | $1,072,897.74 | $18,052,856.64 | $(1,704,148.79) |

| NYSEG - for 1 event in 2019 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $116.06 | $1,683,412.42 | $107,286.96 | $1,637,473.34 | $(61,347.88) |
| 10% | $116.06 | $3,366,940.90 | $214,577.05 | $3,461,404.82 | $(309,040.97) |
| 15% | $116.06 | $5,050,353.32 | $321,867.14 | $5,285,336.30 | $(556,850.12) |
| 25% | $116.06 | $8,417,410.27 | $536,447.31 | $8,933,199.25 | $(1,052,236.29) |
| 50% | $116.06 | $16,834,820.54 | $1,072,897.74 | $18,052,856.64 | $(2,290,933.85) |

| NYSEG - for 1 event in 2020 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $101.35 | $1,470,018.51 | $107,286.96 | $1,637,473.34 | $(274,741.79) |
| 10% | $101.35 | $2,940,138.37 | $214,577.05 | $3,461,404.82 | $(735,843.49) |
| 15% | $101.35 | $4,410,156.89 | $321,867.14 | $5,285,336.30 | $(1,197,046.55) |
| 25% | $101.35 | $7,350,396.60 | $536,447.31 | $8,933,199.25 | $(2,119,249.96) |
| 50% | $101.35 | $14,700,793.21 | $1,072,897.74 | $18,052,856.64 | $(4,424,961.18) |

| NYSEG - for 1 event in 2021 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $161.05 | $2,336,034.31 | $107,286.96 | $1,637,473.34 | $591,274.00 |
| 10% | $161.05 | $4,672,229.66 | $214,577.05 | $3,461,404.82 | $996,247.79 |
| 15% | $161.05 | $7,008,263.97 | $321,867.14 | $5,285,336.30 | $1,401,060.53 |
| 25% | $161.05 | $11,680,654.68 | $536,447.31 | $8,933,199.25 | $2,211,008.11 |
| 50% | $161.05 | $23,361,309.35 | $1,072,897.74 | $18,052,856.64 | $4,235,554.97 |

| NYSEG - for 1 event in 2022 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $166.61 | $2,416,703.33 | $107,286.96 | $1,637,473.34 | $671,943.03 |
| 10% | $166.61 | $4,833,573.27 | $214,577.05 | $3,461,404.82 | $1,157,591.40 |
| 15% | $166.61 | $7,250,276.60 | $321,867.14 | $5,285,336.30 | $1,643,073.17 |
| 25% | $166.61 | $12,084,016.48 | $536,447.31 | $8,933,199.25 | $2,614,369.92 |
| 50% | $166.61 | $24,168,032.96 | $1,072,897.74 | $18,052,856.64 | $5,042,278.58 |

| NYSEG - for 1 event in 2023 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $176.80 | $2,564,556.00 | $107,286.96 | $1,637,473.34 | $819,795.69 |
| 10% | $176.80 | $5,129,288.80 | $214,577.05 | $3,461,404.82 | $1,453,306.93 |
| 15% | $176.80 | $7,693,844.79 | $321,867.14 | $5,285,336.30 | $2,086,641.36 |
| 25% | $176.80 | $12,823,310.39 | $536,447.31 | $8,933,199.25 | $3,353,663.83 |
| 50% | $176.80 | $25,646,620.79 | $1,072,897.74 | $18,052,856.64 | $6,520,866.40 |

| NYSEG - for 1 event in 2024 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $186.36 | $2,703,081.93 | $107,286.96 | $1,637,473.34 | $958,321.63 |
| 10% | $186.36 | $5,406,350.21 | $214,577.05 | $3,461,404.82 | $1,730,368.34 |
| 15% | $186.36 | $8,109,432.14 | $321,867.14 | $5,285,336.30 | $2,502,228.71 |
| 25% | $186.36 | $13,515,968.71 | $536,447.31 | $8,933,199.25 | $4,046,322.14 |
| 50% | $186.36 | $27,031,937.41 | $1,072,897.74 | $18,052,856.64 | $7,906,183.02 |

| NYSEG - for 1 event in 2025 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $199.75 | $2,897,319.04 | $107,286.96 | $1,637,473.34 | $1,152,558.74 |
| 10% | $199.75 | $5,794,837.84 | $214,577.05 | $3,461,404.82 | $2,118,855.97 |
| 15% | $199.75 | $8,692,156.88 | $321,867.14 | $5,285,336.30 | $3,084,953.45 |
| 25% | $199.75 | $14,487,194.46 | $536,447.31 | $8,933,199.25 | $5,017,547.90 |
| 50% | $199.75 | $28,974,388.92 | $1,072,897.74 | $18,052,856.64 | $9,848,634.54 |

| NYSEG - for 1 event in 2026 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $215.07 | $3,119,531.24 | $107,286.96 | $1,637,473.34 | $1,374,770.93 |
| 10% | $215.07 | $6,239,277.54 | $214,577.05 | $3,461,404.82 | $2,563,295.67 |
| 15% | $215.07 | $9,358,808.78 | $321,867.14 | $5,285,336.30 | $3,751,605.35 |
| 25% | $215.07 | $15,598,301.38 | $536,447.31 | $8,933,199.25 | $6,128,654.82 |
| 50% | $215.07 | $31,196,602.77 | $1,072,897.74 | $18,052,856.64 | $12,070,848.38 |

| NYSEG - for 1 event in 2027 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $228.56 | $3,315,279.96 | $107,286.96 | $1,637,473.34 | $1,570,519.66 |
| 10% | $228.56 | $6,630,788.48 | $214,577.05 | $3,461,404.82 | $2,954,806.62 |
| 15% | $228.56 | $9,946,068.44 | $321,867.14 | $5,285,336.30 | $4,338,865.01 |
| 25% | $228.56 | $16,577,085.49 | $536,447.31 | $8,933,199.25 | $7,107,438.93 |
| 50% | $228.56 | $33,154,170.98 | $1,072,897.74 | $18,052,856.64 | $14,028,416.59 |

| RG&E - for 1 event in 2017 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $137.88 | $1,549,940.30 | $63,437.13 | $878,596.15 | $607,907.02 |
| 10% | $137.88 | $3,099,880.60 | $126,874.25 | $1,957,027.30 | $1,015,979.04 |
| 15% | $137.88 | $4,649,820.90 | $190,311.38 | $3,035,458.45 | $1,424,051.06 |
| 25% | $137.88 | $7,749,701.50 | $317,185.63 | $5,192,320.76 | $2,240,195.11 |
| 50% | $137.88 | $15,499,540.87 | $634,371.27 | $10,584,476.51 | $4,280,693.09 |

| RG&E - for 1 event in 2018 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $120.10 | $1,350,073.55 | $63,437.13 | $878,596.15 | $408,040.27 |
| 10% | $120.10 | $2,700,147.10 | $126,874.25 | $1,957,027.30 | $616,245.54 |
| 15% | $120.10 | $4,050,220.64 | $190,311.38 | $3,035,458.45 | $824,450.81 |
| 25% | $120.10 | $6,750,367.74 | $317,185.63 | $5,192,320.76 | $1,240,861.35 |
| 50% | $120.10 | $13,500,855.58 | $634,371.27 | $10,584,476.51 | $2,282,007.80 |

| RG&E - for 1 event in 2019 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $116.06 | $1,304,601.10 | $63,437.13 | $878,596.15 | $362,567.83 |
| 10% | $116.06 | $2,609,202.21 | $126,874.25 | $1,957,027.30 | $525,300.65 |
| 15% | $116.06 | $3,913,803.31 | $190,311.38 | $3,035,458.45 | $688,033.48 |
| 25% | $116.06 | $6,523,005.52 | $317,185.63 | $5,192,320.76 | $1,013,499.13 |
| 50% | $116.06 | $13,046,127.10 | $634,371.27 | $10,584,476.51 | $1,827,279.32 |

| RG&E - for 1 event in 2020 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $101.35 | $1,139,226.34 | $63,437.13 | $878,596.15 | $197,193.07 |
| 10% | $101.35 | $2,278,452.69 | $126,874.25 | $1,957,027.30 | $194,551.13 |
| 15% | $101.35 | $3,417,679.03 | $190,311.38 | $3,035,458.45 | $191,909.20 |
| 25% | $101.35 | $5,696,131.72 | $317,185.63 | $5,192,320.76 | $186,625.33 |
| 50% | $101.35 | $11,392,364.78 | $634,371.27 | $10,584,476.51 | $173,517.00 |

| RG&E - for 1 event in 2021 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $161.05 | $1,810,366.19 | $63,437.13 | $878,596.15 | $868,332.91 |
| 10% | $161.05 | $3,620,732.39 | $126,874.25 | $1,957,027.30 | $1,536,830.83 |
| 15% | $161.05 | $5,431,098.58 | $190,311.38 | $3,035,458.45 | $2,205,328.74 |
| 25% | $161.05 | $9,051,830.96 | $317,185.63 | $5,192,320.76 | $3,542,324.57 |
| 50% | $161.05 | $18,103,822.98 | $634,371.27 | $10,584,476.51 | $6,884,975.20 |

| RG&E - for 1 event in 2022 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $166.61 | $1,872,882.60 | $63,437.13 | $878,596.15 | $930,849.32 |
| 10% | $166.61 | $3,745,765.20 | $126,874.25 | $1,957,027.30 | $1,661,863.65 |
| 15% | $166.61 | $5,618,647.80 | $190,311.38 | $3,035,458.45 | $2,392,877.97 |
| 25% | $166.61 | $9,364,413.00 | $317,185.63 | $5,192,320.76 | $3,854,906.61 |
| 50% | $166.61 | $18,728,992.62 | $634,371.27 | $10,584,476.51 | $7,510,144.84 |

| RG&E - for 1 event in 2023 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $176.80 | $1,987,464.59 | $63,437.13 | $878,596.15 | $1,045,431.32 |
| 10% | $176.80 | $3,974,929.19 | $126,874.25 | $1,957,027.30 | $1,891,027.63 |
| 15% | $176.80 | $5,962,393.78 | $190,311.38 | $3,035,458.45 | $2,736,623.95 |
| 25% | $176.80 | $9,937,322.97 | $317,185.63 | $5,192,320.76 | $4,427,816.58 |
| 50% | $176.80 | $19,874,822.75 | $634,371.27 | $10,584,476.51 | $8,655,974.97 |

| RG&E - for 1 event in 2024 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $186.36 | $2,094,818.61 | $63,437.13 | $878,596.15 | $1,152,785.33 |
| 10% | $186.36 | $4,189,637.22 | $126,874.25 | $1,957,027.30 | $2,105,735.67 |
| 15% | $186.36 | $6,284,455.83 | $190,311.38 | $3,035,458.45 | $3,058,686.00 |
| 25% | $186.36 | $10,474,093.06 | $317,185.63 | $5,192,320.76 | $4,964,586.67 |
| 50% | $186.36 | $20,948,372.47 | $634,371.27 | $10,584,476.51 | $9,729,524.69 |

| RG&E - for 1 event in 2025 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $199.75 | $2,245,347.35 | $63,437.13 | $878,596.15 | $1,303,314.08 |
| 10% | $199.75 | $4,490,694.71 | $126,874.25 | $1,957,027.30 | $2,406,793.15 |
| 15% | $199.75 | $6,736,042.06 | $190,311.38 | $3,035,458.45 | $3,510,272.23 |
| 25% | $199.75 | $11,226,736.77 | $317,185.63 | $5,192,320.76 | $5,717,230.38 |
| 50% | $199.75 | $22,453,673.29 | $634,371.27 | $10,584,476.51 | $11,234,825.51 |

| RG&E - for 1 event in 2026 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $215.07 | $2,417,556.06 | $63,437.13 | $878,596.15 | $1,475,522.78 |
| 10% | $215.07 | $4,835,112.12 | $126,874.25 | $1,957,027.30 | $2,751,210.56 |
| 15% | $215.07 | $7,252,668.18 | $190,311.38 | $3,035,458.45 | $4,026,898.34 |
| 25% | $215.07 | $12,087,780.30 | $317,185.63 | $5,192,320.76 | $6,578,273.91 |
| 50% | $215.07 | $24,175,775.66 | $634,371.27 | $10,584,476.51 | $12,956,927.88 |

| RG&E - for 1 event in 2027 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $228.56 | $2,569,256.26 | $63,437.13 | $878,596.15 | $1,627,222.98 |
| 10% | $228.56 | $5,138,512.52 | $126,874.25 | $1,957,027.30 | $3,054,610.96 |
| 15% | $228.56 | $7,707,768.78 | $190,311.38 | $3,035,458.45 | $4,481,998.94 |
| 25% | $228.56 | $12,846,281.30 | $317,185.63 | $5,192,320.76 | $7,336,774.91 |
| 50% | $228.56 | $25,692,791.16 | $634,371.27 | $10,584,476.51 | $14,473,943.38 |

| National Grid - for 1 event in 2017 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $137.88 | $7,792,031.51 | $363,888.00 | $6,186,096.00 | $1,242,047.51 |
| 10% | $137.88 | $15,584,063.02 | $1,039,675.00 | $12,372,132.50 | $2,172,255.52 |
| 15% | $137.88 | $23,376,094.52 | $1,559,515.00 | $18,558,228.50 | $3,258,351.02 |
| 25% | $137.88 | $38,960,019.66 | $2,599,190.00 | $30,930,361.00 | $5,430,468.66 |
| 50% | $137.88 | $77,920,039.31 | $5,198,380.00 | $61,860,722.00 | $10,860,937.31 |

| National Grid - for 1 event in 2018 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $120.10 | $6,787,239.24 | $363,888.00 | $6,186,096.00 | $237,255.24 |
| 10% | $120.10 | $13,574,478.49 | $1,039,675.00 | $12,372,132.50 | $162,670.99 |
| 15% | $120.10 | $20,361,717.73 | $1,559,515.00 | $18,558,228.50 | $243,974.23 |
| 25% | $120.10 | $33,936,076.11 | $2,599,190.00 | $30,930,361.00 | $406,525.11 |
| 50% | $120.10 | $67,872,152.22 | $5,198,380.00 | $61,860,722.00 | $813,050.22 |

| National Grid - for 1 event in 2019 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $116.06 | $6,558,635.14 | $363,888.00 | $6,186,096.00 | $8,651.14 |
| 10% | $116.06 | $13,117,270.27 | $1,039,675.00 | $12,372,132.50 | $(294,537.23) |
| 15% | $116.06 | $19,675,905.41 | $1,559,515.00 | $18,558,228.50 | $(441,838.09) |
| 25% | $116.06 | $32,793,059.63 | $2,599,190.00 | $30,930,361.00 | $(736,491.37) |
| 50% | $116.06 | $65,586,119.26 | $5,198,380.00 | $61,860,722.00 | $(1,472,982.74) |

| National Grid - for 1 event in 2020 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $101.35 | $5,727,244.83 | $363,888.00 | $6,186,096.00 | $(822,739.17) |
| 10% | $101.35 | $11,454,489.66 | $1,039,675.00 | $12,372,132.50 | $(1,957,317.84) |
| 15% | $101.35 | $17,181,734.49 | $1,559,515.00 | $18,558,228.50 | $(2,936,009.01) |
| 25% | $101.35 | $5,696,131.72 | $2,599,190.00 | $30,930,361.00 | $(27,833,419.28) |
| 50% | $101.35 | $11,392,364.78 | $5,198,380.00 | $61,860,722.00 | $(55,666,737.22) |

| National Grid - for 1 event in 2021 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $161.05 | $9,101,273.40 | $363,888.00 | $6,186,096.00 | $2,551,289.40 |
| 10% | $161.05 | $18,202,546.80 | $1,039,675.00 | $12,372,132.50 | $4,790,739.30 |
| 15% | $161.05 | $27,303,820.20 | $1,559,515.00 | $18,558,228.50 | $7,186,076.70 |
| 25% | $161.05 | $45,506,205.94 | $2,599,190.00 | $30,930,361.00 | $11,976,654.94 |
| 50% | $161.05 | $91,012,411.89 | $5,198,380.00 | $61,860,722.00 | $23,953,309.89 |

| National Grid - for 1 event in 2022 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $166.61 | $9,415,562.81 | $363,888.00 | $6,186,096.00 | $2,865,578.81 |
| 10% | $166.61 | $18,831,125.62 | $1,039,675.00 | $12,372,132.50 | $5,419,318.12 |
| 15% | $166.61 | $28,246,688.43 | $1,559,515.00 | $18,558,228.50 | $8,128,944.93 |
| 25% | $166.61 | $47,077,647.43 | $2,599,190.00 | $30,930,361.00 | $13,548,096.43 |
| 50% | $166.61 | $94,155,294.86 | $5,198,380.00 | $61,860,722.00 | $27,096,192.86 |

| National Grid - for 1 event in 2023 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $176.80 | $9,991,602.10 | $363,888.00 | $6,186,096.00 | $3,441,618.10 |
| 10% | $176.80 | $19,983,204.20 | $1,039,675.00 | $12,372,132.50 | $6,571,396.70 |
| 15% | $176.80 | $29,974,806.29 | $1,559,515.00 | $18,558,228.50 | $9,857,062.79 |
| 25% | $176.80 | $49,957,833.68 | $2,599,190.00 | $30,930,361.00 | $16,428,282.68 |
| 50% | $176.80 | $99,915,667.37 | $5,198,380.00 | $61,860,722.00 | $32,856,565.37 |

| National Grid - for 1 event in 2024 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $186.36 | $10,531,304.10 | $363,888.00 | $6,186,096.00 | $3,981,320.10 |
| 10% | $186.36 | $21,062,608.19 | $1,039,675.00 | $12,372,132.50 | $7,650,800.69 |
| 15% | $186.36 | $31,593,912.29 | $1,559,515.00 | $18,558,228.50 | $11,476,168.79 |
| 25% | $186.36 | $52,656,334.13 | $2,599,190.00 | $30,930,361.00 | $19,126,783.13 |
| 50% | $186.36 | $105,312,668.25 | $5,198,380.00 | $61,860,722.00 | $38,253,566.25 |

| National Grid - for 1 event in 2025 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $199.75 | $11,288,058.87 | $363,888.00 | $6,186,096.00 | $4,738,074.87 |
| 10% | $199.75 | $22,576,117.73 | $1,039,675.00 | $12,372,132.50 | $9,164,310.23 |
| 15% | $199.75 | $33,864,176.60 | $1,559,515.00 | $18,558,228.50 | $13,746,433.10 |
| 25% | $199.75 | $56,440,094.58 | $2,599,190.00 | $30,930,361.00 | $22,910,543.58 |
| 50% | $199.75 | $112,880,189.16 | $5,198,380.00 | $61,860,722.00 | $45,821,087.16 |

| National Grid - for 1 event in 2026 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $215.07 | $12,153,805.53 | $363,888.00 | $6,186,096.00 | $5,603,821.53 |
| 10% | $215.07 | $24,307,611.07 | $1,039,675.00 | $12,372,132.50 | $10,895,803.57 |
| 15% | $215.07 | $36,461,416.60 | $1,559,515.00 | $18,558,228.50 | $16,343,673.10 |
| 25% | $215.07 | $60,768,812.60 | $2,599,190.00 | $30,930,361.00 | $27,239,261.60 |
| 50% | $215.07 | $121,537,625.21 | $5,198,380.00 | $61,860,722.00 | $54,478,523.21 |

| National Grid - for 1 event in 2027 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Initial Cost | Economic Benefit |
| 5% | $228.56 | $12,916,449.58 | $363,888.00 | $6,186,096.00 | $6,366,465.58 |
| 10% | $228.56 | $25,832,899.16 | $1,039,675.00 | $12,372,132.50 | $12,421,091.66 |
| 15% | $228.56 | $38,749,348.74 | $1,559,515.00 | $18,558,228.50 | $18,631,605.24 |
| 25% | $228.56 | $64,582,019.34 | $2,599,190.00 | $30,930,361.00 | $31,052,468.34 |
| 50% | $228.56 | $129,164,038.68 | $5,198,380.00 | $61,860,722.00 | $62,104,936.68 |

BYOT and DIT Programs: Impact of Energy Efficiency

| NYSEG - for 1 event |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 13,780 | $120.10 | $1,654,984.07 | $(89,776.23) |
| 2019 | 13,780 | $116.06 | $1,599,241.80 | $(145,518.50) |
| 2020 | 13,780 | $101.35 | $1,396,517.59 | $(348,242.71) |
| 2021 | 13,780 | $161.05 | $2,219,232.59 | $474,472.29 |
| 2022 | 13,780 | $166.61 | $2,295,868.16 | $551,107.86 |
| 2023 | 13,780 | $176.80 | $2,436,328.20 | $691,567.89 |
| 2024 | 13,780 | $186.36 | $2,567,927.83 | $823,167.53 |
| 2025 | 13,780 | $199.75 | $2,752,453.09 | $1,007,692.79 |
| 2026 | 13,780 | $215.07 | $2,963,554.68 | $1,218,794.37 |
| 2027 | 13,780 | $228.56 | $3,149,515.96 | $1,404,755.66 |

| NYSEG - for 1 event |  |  |  |  |
| --- | --- | --- | --- | --- |
| 10% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 13,055 | $120.10 | $1,567,879.65 | $(176,880.65) |
| 2019 | 13,055 | $116.06 | $1,515,071.18 | $(229,689.12) |
| 2020 | 13,055 | $101.35 | $1,323,016.66 | $(421,743.64) |
| 2021 | 13,055 | $161.05 | $2,102,430.87 | $357,670.57 |
| 2022 | 13,055 | $166.61 | $2,175,033.00 | $430,272.69 |
| 2023 | 13,055 | $176.80 | $2,308,100.40 | $563,340.09 |
| 2024 | 13,055 | $186.36 | $2,432,773.74 | $688,013.43 |
| 2025 | 13,055 | $199.75 | $2,607,587.14 | $862,826.84 |
| 2026 | 13,055 | $215.07 | $2,807,578.11 | $1,062,817.81 |
| 2027 | 13,055 | $228.56 | $2,983,751.97 | $1,238,991.66 |

| NYSEG - for 1 event |  |  |  |  |
| --- | --- | --- | --- | --- |
| 15% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 12,329 | $120.10 | $1,480,775.22 | $(263,985.08) |
| 2019 | 12,329 | $116.06 | $1,430,900.56 | $(313,859.75) |
| 2020 | 12,329 | $101.35 | $1,249,515.74 | $(495,244.57) |
| 2021 | 12,329 | $161.05 | $1,985,629.16 | $240,868.86 |
| 2022 | 12,329 | $166.61 | $2,054,197.83 | $309,437.53 |
| 2023 | 12,329 | $176.80 | $2,179,872.60 | $435,112.29 |
| 2024 | 12,329 | $186.36 | $2,297,619.64 | $552,859.34 |
| 2025 | 12,329 | $199.75 | $2,462,721.19 | $717,960.89 |
| 2026 | 12,329 | $215.07 | $2,651,601.55 | $906,841.25 |
| 2027 | 12,329 | $228.56 | $2,817,987.97 | $1,073,227.66 |

| RG&E |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 10,679 | $120.10 | $1,282,569.87 | $340,536.59 |
| 2019 | 10,679 | $116.06 | $1,239,371.05 | $297,337.77 |
| 2020 | 10,679 | $101.35 | $1,082,265.03 | $140,231.75 |
| 2021 | 10,679 | $161.05 | $1,719,847.88 | $777,814.61 |
| 2022 | 10,679 | $166.61 | $1,779,238.47 | $837,205.19 |
| 2023 | 10,679 | $176.80 | $1,888,091.37 | $946,058.09 |
| 2024 | 10,679 | $186.36 | $1,990,077.68 | $1,048,044.40 |
| 2025 | 10,679 | $199.75 | $2,133,079.99 | $1,191,046.71 |
| 2026 | 10,679 | $215.07 | $2,296,678.26 | $1,354,644.98 |
| 2027 | 10,679 | $228.56 | $2,440,793.45 | $1,498,760.17 |

| RG&E |  |  |  |  |
| --- | --- | --- | --- | --- |
| 10% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 10,117 | $120.10 | $1,215,066.19 | $273,032.92 |
| 2019 | 10,117 | $116.06 | $1,174,140.99 | $232,107.72 |
| 2020 | 10,117 | $101.35 | $1,025,303.71 | $83,270.43 |
| 2021 | 10,117 | $161.05 | $1,629,329.57 | $687,296.30 |
| 2022 | 10,117 | $166.61 | $1,685,594.34 | $743,561.06 |
| 2023 | 10,117 | $176.80 | $1,788,718.14 | $846,684.86 |
| 2024 | 10,117 | $186.36 | $1,885,336.75 | $943,303.47 |
| 2025 | 10,117 | $199.75 | $2,020,812.62 | $1,078,779.34 |
| 2026 | 10,117 | $215.07 | $2,175,800.45 | $1,233,767.18 |
| 2027 | 10,117 | $228.56 | $2,312,330.63 | $1,370,297.36 |

| RG&E |  |  |  |  |
| --- | --- | --- | --- | --- |
| 15% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 9,555 | $120.10 | $1,147,562.52 | $205,529.24 |
| 2019 | 9,555 | $116.06 | $1,108,910.94 | $166,877.66 |
| 2020 | 9,555 | $101.35 | $968,342.39 | $26,309.11 |
| 2021 | 9,555 | $161.05 | $1,538,811.26 | $596,777.99 |
| 2022 | 9,555 | $166.61 | $1,591,950.21 | $649,916.93 |
| 2023 | 9,555 | $176.80 | $1,689,344.91 | $747,311.63 |
| 2024 | 9,555 | $186.36 | $1,780,595.82 | $838,562.54 |
| 2025 | 9,555 | $199.75 | $1,908,545.25 | $966,511.97 |
| 2026 | 9,555 | $215.07 | $2,054,922.65 | $1,112,889.37 |
| 2027 | 9,555 | $228.56 | $2,183,867.82 | $1,241,834.54 |

| National Grid |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 53,686 | $120.10 | $6,447,877.28 | $(102,106.72) |
| 2019 | 53,686 | $116.06 | $6,230,703.38 | $(319,280.62) |
| 2020 | 53,686 | $101.35 | $5,440,882.59 | $(1,109,101.41) |
| 2021 | 53,686 | $161.05 | $8,646,209.73 | $2,096,225.73 |
| 2022 | 53,686 | $166.61 | $8,944,784.67 | $2,394,800.67 |
| 2023 | 53,686 | $176.80 | $9,492,021.99 | $2,942,037.99 |
| 2024 | 53,686 | $186.36 | $10,004,738.89 | $3,454,754.89 |
| 2025 | 53,686 | $199.75 | $10,723,655.92 | $4,173,671.92 |
| 2026 | 53,686 | $215.07 | $11,546,115.26 | $4,996,131.26 |
| 2027 | 53,686 | $228.56 | $12,270,627.10 | $5,720,643.10 |

| National Grid |  |  |  |  |
| --- | --- | --- | --- | --- |
| 10% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 50,861 | $120.10 | $6,108,515.32 | $(441,468.68) |
| 2019 | 50,861 | $116.06 | $5,902,771.62 | $(647,212.38) |
| 2020 | 50,861 | $101.35 | $5,154,520.35 | $(1,395,463.65) |
| 2021 | 50,861 | $161.05 | $8,191,146.06 | $1,641,162.06 |
| 2022 | 50,861 | $166.61 | $8,474,006.53 | $1,924,022.53 |
| 2023 | 50,861 | $176.80 | $8,992,441.89 | $2,442,457.89 |
| 2024 | 50,861 | $186.36 | $9,478,173.69 | $2,928,189.69 |
| 2025 | 50,861 | $199.75 | $10,159,252.98 | $3,609,268.98 |
| 2026 | 50,861 | $215.07 | $10,938,424.98 | $4,388,440.98 |
| 2027 | 50,861 | $228.56 | $11,624,804.62 | $5,074,820.62 |

| National Grid |  |  |  |  |
| --- | --- | --- | --- | --- |
| 15% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 48,035 | $120.10 | $5,769,153.36 | $(780,830.64) |
| 2019 | 48,035 | $116.06 | $5,574,839.87 | $(975,144.13) |
| 2020 | 48,035 | $101.35 | $4,868,158.10 | $(1,681,825.90) |
| 2021 | 48,035 | $161.05 | $7,736,082.39 | $1,186,098.39 |
| 2022 | 48,035 | $166.61 | $8,003,228.39 | $1,453,244.39 |
| 2023 | 48,035 | $176.80 | $8,492,861.78 | $1,942,877.78 |
| 2024 | 48,035 | $186.36 | $8,951,608.48 | $2,401,624.48 |
| 2025 | 48,035 | $199.75 | $9,594,850.04 | $3,044,866.04 |
| 2026 | 48,035 | $215.07 | $10,330,734.70 | $3,780,750.70 |
| 2027 | 48,035 | $228.56 | $10,978,982.14 | $4,428,998.14 |

BYOT and DIT Programs: Summary

For NYSEG, these programs are economically viable for the years 2017, 2021, 2022, 2023, 2024, 2025, 2026, and 2027. The economic benefit could be increased by either increasing the number of events per year, increasing the duration of the events, reducing the number of participants, or by targeting residents within zones of higher AGCC. NYSEG participants reduce an average of 0.68 kW per event per participant. For all levels of increased energy efficiency (5, 10, and 15 percent), these programs are economically viable for the years 2021 through 2027.

For RG&E, these programs were economically viable for all years. This is due to the fact that RG&E participants reduce an average of 0.89 kW per event per participant. They also have less initial cost as they have less potential participants. For all levels of increased energy efficiency (5, 10, and 15 percent), these programs are economically viable for all years.

If National Grid were to incorporate these programs and to achieve an average reduction of 0.78 kW per event per participant, they would be economically viable for all years except 2020. The economic benefit could be increased by either increasing the number of events per year, increasing the duration of the events, reducing the number of participants, or by targeting residents within zones of higher AGCC. For all levels of increased energy efficiency (5, 10, and 15 percent), these programs are economically viable for the years 2021 through 2027.

RoomAC Program

| NYSEG |  |  |
| --- | --- | --- |
| Enrollment | Residential Count | Estimated kW Relief per Event |
| 5% | 34,324 | 3.86 |
| 10% | 68,649 | 7.72 |
| 15% | 102,974 | 11.58 |
| 25% | 171,624 | 19.30 |
| 50% | 343,249 | 38.60 |

| RG&E |  |  |
| --- | --- | --- |
| Enrollment | Residential Count | Estimated kW Relief per Event |
| 5% | 16,546 | 1.93 |
| 10% | 33,092 | 3.86 |
| 15% | 49,638 | 5.79 |
| 25% | 82,730 | 9.65 |
| 50% | 165,460 | 19.29 |

| National Grid |  |  |
| --- | --- | --- |
| Enrollment | Residential Count | Estimated kW Relief per Event |
| 5% | 103,968 | 12.48 |
| 10% | 207,935 | 24.95 |
| 15% | 311,903 | 37.43 |
| 25% | 519,838 | 62.38 |
| 50% | 1,039,676 | 124.76 |

| Event Description | - Typically, the utility comes close to peak load about 4 or 5 times a year for about 3 to 4 hours. When this happens, the utility would notify the participant and, if necessary, could manually raise their air conditioner temperature setting by a few degrees to reduce the overall peak load. Participants are able to manually override this higher setting. |
| --- | --- |
| Requirements | - Participants must install the smartAC kit. |
| Incentives | - Participants receive a smartAC kit, which includes a remote control, instructions on how to set it up, and a $25 gift card. |

Cost

Initial (per participant)

$165

| NYSEG - for 4 events in 2017 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $137.88 | $2,128.91 | $1,905,899.76 | $(1,903,770.85) |
| 10% | $137.88 | $4,257.82 | $4,028,823.22 | $(4,024,565.40) |
| 15% | $137.88 | $6,386.73 | $6,151,746.69 | $(6,145,359.96) |
| 25% | $137.88 | $10,644.55 | $10,397,593.61 | $(10,386,949.06) |
| 50% | $137.88 | $21,289.10 | $21,012,210.91 | $(20,990,921.81) |

| NYSEG - for 4 events in 2018 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $120.10 | $1,854.38 | $1,905,899.76 | $(1,904,045.38) |
| 10% | $120.10 | $3,708.77 | $4,028,823.22 | $(4,025,114.46) |
| 15% | $120.10 | $5,563.15 | $6,151,746.69 | $(6,146,183.53) |
| 25% | $120.10 | $9,271.92 | $10,397,593.61 | $(10,388,321.69) |
| 50% | $120.10 | $18,543.84 | $21,012,210.91 | $(20,993,667.07) |

| NYSEG - for 4 events in 2019 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $116.06 | $1,791.93 | $1,905,899.76 | $(1,904,107.84) |
| 10% | $116.06 | $3,583.85 | $4,028,823.22 | $(4,025,239.37) |
| 15% | $116.06 | $5,375.78 | $6,151,746.69 | $(6,146,370.91) |
| 25% | $116.06 | $8,959.63 | $10,397,593.61 | $(10,388,633.98) |
| 50% | $116.06 | $17,919.26 | $21,012,210.91 | $(20,994,291.65) |

| NYSEG - for 4 events in 2020 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $101.35 | $1,564.78 | $1,905,899.76 | $(1,904,334.99) |
| 10% | $101.35 | $3,129.55 | $4,028,823.22 | $(4,025,693.67) |
| 15% | $101.35 | $4,694.33 | $6,151,746.69 | $(6,147,052.36) |
| 25% | $101.35 | $7,823.88 | $10,397,593.61 | $(10,389,769.72) |
| 50% | $101.35 | $15,647.77 | $21,012,210.91 | $(20,996,563.15) |

| NYSEG - for 4 events in 2021 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $161.05 | $2,486.62 | $1,905,899.76 | $(1,903,413.15) |
| 10% | $161.05 | $4,973.23 | $4,028,823.22 | $(4,023,849.99) |
| 15% | $161.05 | $7,459.85 | $6,151,746.69 | $(6,144,286.84) |
| 25% | $161.05 | $12,433.08 | $10,397,593.61 | $(10,385,160.53) |
| 50% | $161.05 | $24,866.16 | $21,012,210.91 | $(20,987,344.75) |

| NYSEG - for 4 events in 2022 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $166.61 | $2,572.49 | $1,905,899.76 | $(1,903,327.28) |
| 10% | $166.61 | $5,144.97 | $4,028,823.22 | $(4,023,678.25) |
| 15% | $166.61 | $7,717.46 | $6,151,746.69 | $(6,144,029.23) |
| 25% | $166.61 | $12,862.43 | $10,397,593.61 | $(10,384,731.18) |
| 50% | $166.61 | $25,724.85 | $21,012,210.91 | $(20,986,486.06) |

| NYSEG - for 4 events in 2023 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $176.80 | $2,729.87 | $1,905,899.76 | $(1,903,169.89) |
| 10% | $176.80 | $5,459.74 | $4,028,823.22 | $(4,023,363.49) |
| 15% | $176.80 | $8,189.61 | $6,151,746.69 | $(6,143,557.08) |
| 25% | $176.80 | $13,649.34 | $10,397,593.61 | $(10,383,944.26) |
| 50% | $176.80 | $27,298.69 | $21,012,210.91 | $(20,984,912.23) |

| NYSEG - for 4 events in 2024 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $186.36 | $2,877.32 | $1,905,899.76 | $(1,903,022.44) |
| 10% | $186.36 | $5,754.65 | $4,028,823.22 | $(4,023,068.58) |
| 15% | $186.36 | $8,631.97 | $6,151,746.69 | $(6,143,114.71) |
| 25% | $186.36 | $14,386.62 | $10,397,593.61 | $(10,383,206.99) |
| 50% | $186.36 | $28,773.24 | $21,012,210.91 | $(20,983,437.67) |

| NYSEG - for 4 events in 2025 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $199.75 | $3,084.08 | $1,905,899.76 | $(1,902,815.68) |
| 10% | $199.75 | $6,168.16 | $4,028,823.22 | $(4,022,655.06) |
| 15% | $199.75 | $9,252.25 | $6,151,746.69 | $(6,142,494.44) |
| 25% | $199.75 | $15,420.41 | $10,397,593.61 | $(10,382,173.20) |
| 50% | $199.75 | $30,840.82 | $21,012,210.91 | $(20,981,370.09) |

| NYSEG - for 4 events in 2026 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $215.07 | $3,320.62 | $1,905,899.76 | $(1,902,579.15) |
| 10% | $215.07 | $6,641.24 | $4,028,823.22 | $(4,022,181.99) |
| 15% | $215.07 | $9,961.85 | $6,151,746.69 | $(6,141,784.83) |
| 25% | $215.07 | $16,603.09 | $10,397,593.61 | $(10,380,990.52) |
| 50% | $215.07 | $33,206.18 | $21,012,210.91 | $(20,979,004.73) |

| NYSEG - for 4 events in 2027 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $228.56 | $3,528.98 | $1,905,899.76 | $(1,902,370.78) |
| 10% | $228.56 | $7,057.97 | $4,028,823.22 | $(4,021,765.25) |
| 15% | $228.56 | $10,586.95 | $6,151,746.69 | $(6,141,159.73) |
| 25% | $228.56 | $17,644.92 | $10,397,593.61 | $(10,379,948.68) |
| 50% | $228.56 | $35,289.85 | $21,012,210.91 | $(20,976,921.07) |

| RG&E - for 4 events in 2017 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $137.88 | $1,064.46 | $518,658.39 | $(517,593.93) |
| 10% | $137.88 | $2,128.91 | $1,155,284.63 | $(1,153,155.72) |
| 15% | $137.88 | $3,193.37 | $1,791,910.86 | $(1,788,717.50) |
| 25% | $137.88 | $5,322.28 | $3,065,163.34 | $(3,059,841.07) |
| 50% | $137.88 | $10,639.04 | $6,248,294.53 | $(6,237,655.50) |

| RG&E - for 4 events in 2018 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $120.10 | $927.19 | $518,658.39 | $(517,731.20) |
| 10% | $120.10 | $1,854.38 | $1,155,284.63 | $(1,153,430.24) |
| 15% | $120.10 | $2,781.58 | $1,791,910.86 | $(1,789,129.29) |
| 25% | $120.10 | $4,635.96 | $3,065,163.34 | $(3,060,527.38) |
| 50% | $120.10 | $9,267.12 | $6,248,294.53 | $(6,239,027.41) |

| RG&E - for 4 events in 2019 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $116.06 | $895.96 | $518,658.39 | $(517,762.43) |
| 10% | $116.06 | $1,791.93 | $1,155,284.63 | $(1,153,492.70) |
| 15% | $116.06 | $2,687.89 | $1,791,910.86 | $(1,789,222.98) |
| 25% | $116.06 | $4,479.82 | $3,065,163.34 | $(3,060,683.53) |
| 50% | $116.06 | $8,954.99 | $6,248,294.53 | $(6,239,339.54) |

| RG&E - for 4 events in 2020 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $101.35 | $782.39 | $518,658.39 | $(517,876.00) |
| 10% | $101.35 | $1,564.78 | $1,155,284.63 | $(1,153,719.85) |
| 15% | $101.35 | $2,347.17 | $1,791,910.86 | $(1,789,563.70) |
| 25% | $101.35 | $3,911.94 | $3,065,163.34 | $(3,061,251.40) |
| 50% | $101.35 | $7,819.83 | $6,248,294.53 | $(6,240,474.70) |

| RG&E - for 4 events in 2021 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $161.05 | $1,243.31 | $518,658.39 | $(517,415.08) |
| 10% | $161.05 | $2,486.62 | $1,155,284.63 | $(1,152,798.01) |
| 15% | $161.05 | $3,729.92 | $1,791,910.86 | $(1,788,180.94) |
| 25% | $161.05 | $6,216.54 | $3,065,163.34 | $(3,058,946.80) |
| 50% | $161.05 | $12,426.64 | $6,248,294.53 | $(6,235,867.89) |

| RG&E - for 4 events in 2022 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $166.61 | $2,572.49 | $518,658.39 | $(516,085.90) |
| 10% | $166.61 | $5,144.97 | $1,155,284.63 | $(1,150,139.66) |
| 15% | $166.61 | $7,717.46 | $1,791,910.86 | $(1,784,193.41) |
| 25% | $166.61 | $12,862.43 | $3,065,163.34 | $(3,052,300.91) |
| 50% | $166.61 | $25,724.85 | $6,248,294.53 | $(6,222,569.68) |

| RG&E - for 4 events in 2023 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $176.80 | $2,729.87 | $518,658.39 | $(515,928.52) |
| 10% | $176.80 | $5,459.74 | $1,155,284.63 | $(1,149,824.89) |
| 15% | $176.80 | $8,189.61 | $1,791,910.86 | $(1,783,721.26) |
| 25% | $176.80 | $13,649.34 | $3,065,163.34 | $(3,051,514.00) |
| 50% | $176.80 | $27,298.69 | $6,248,294.53 | $(6,220,995.84) |

| RG&E - for 4 events in 2024 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $186.36 | $1,438.66 | $518,658.39 | $(517,219.73) |
| 10% | $186.36 | $2,877.32 | $1,155,284.63 | $(1,152,407.30) |
| 15% | $186.36 | $4,315.99 | $1,791,910.86 | $(1,787,594.88) |
| 25% | $186.36 | $7,193.31 | $3,065,163.34 | $(3,057,970.03) |
| 50% | $186.36 | $14,379.17 | $6,248,294.53 | $(6,233,915.37) |

| RG&E - for 4 events in 2025 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $199.75 | $1,542.04 | $518,658.39 | $(517,116.35) |
| 10% | $199.75 | $3,084.08 | $1,155,284.63 | $(1,152,200.54) |
| 15% | $199.75 | $4,626.12 | $1,791,910.86 | $(1,787,284.74) |
| 25% | $199.75 | $7,710.20 | $3,065,163.34 | $(3,057,453.14) |
| 50% | $199.75 | $15,412.42 | $6,248,294.53 | $(6,232,882.11) |

| RG&E - for 4 events in 2026 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $215.07 | $1,660.31 | $518,658.39 | $(516,998.08) |
| 10% | $215.07 | $3,320.62 | $1,155,284.63 | $(1,151,964.01) |
| 15% | $215.07 | $4,980.93 | $1,791,910.86 | $(1,786,929.94) |
| 25% | $215.07 | $8,301.54 | $3,065,163.34 | $(3,056,861.80) |
| 50% | $215.07 | $16,594.49 | $6,248,294.53 | $(6,231,700.04) |

| RG&E - for 4 events in 2027 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $228.56 | $1,764.49 | $518,658.39 | $(516,893.90) |
| 10% | $228.56 | $3,528.98 | $1,155,284.63 | $(1,151,755.64) |
| 15% | $228.56 | $5,293.48 | $1,791,910.86 | $(1,786,617.39) |
| 25% | $228.56 | $8,822.46 | $3,065,163.34 | $(3,056,340.88) |
| 50% | $228.56 | $17,635.78 | $6,248,294.53 | $(6,230,658.75) |

| National Grid - for 4 events in 2017 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $137.88 | $1,720.78 | $701,009.51 | $(699,288.73) |
| 10% | $137.88 | $3,440.18 | $1,519,986.87 | $(1,516,546.70) |
| 15% | $137.88 | $5,160.95 | $2,338,964.24 | $(2,333,803.28) |
| 25% | $137.88 | $8,601.13 | $3,976,918.96 | $(3,968,317.83) |
| 50% | $137.88 | $17,202.26 | $8,071,805.77 | $(8,054,603.51) |

| National Grid - for 4 events in 2018 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $120.10 | $1,498.88 | $701,009.51 | $(699,510.63) |
| 10% | $120.10 | $2,996.56 | $1,519,986.87 | $(1,516,990.31) |
| 15% | $120.10 | $4,495.44 | $2,338,964.24 | $(2,334,468.79) |
| 25% | $120.10 | $7,492.00 | $3,976,918.96 | $(3,969,426.96) |
| 50% | $120.10 | $14,984.00 | $8,071,805.77 | $(8,056,821.77) |

| National Grid - for 4 events in 2019 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $116.06 | $1,448.40 | $701,009.51 | $(699,561.12) |
| 10% | $116.06 | $2,895.63 | $1,519,986.87 | $(1,517,091.24) |
| 15% | $116.06 | $4,344.03 | $2,338,964.24 | $(2,334,620.21) |
| 25% | $116.06 | $7,239.66 | $3,976,918.96 | $(3,969,679.30) |
| 50% | $116.06 | $14,479.32 | $8,071,805.77 | $(8,057,326.45) |

| National Grid - for 4 events in 2020 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $101.35 | $1,264.79 | $701,009.51 | $(699,744.72) |
| 10% | $101.35 | $2,528.57 | $1,519,986.87 | $(1,517,458.30) |
| 15% | $101.35 | $3,793.37 | $2,338,964.24 | $(2,335,170.87) |
| 25% | $101.35 | $6,321.94 | $3,976,918.96 | $(3,970,597.02) |
| 50% | $101.35 | $12,643.88 | $8,071,805.77 | $(8,059,161.89) |

| National Grid - for 4 events in 2021 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $161.05 | $2,009.91 | $701,009.51 | $(698,999.60) |
| 10% | $161.05 | $4,018.20 | $1,519,986.87 | $(1,515,968.67) |
| 15% | $161.05 | $6,028.11 | $2,338,964.24 | $(2,332,936.12) |
| 25% | $161.05 | $10,046.32 | $3,976,918.96 | $(3,966,872.64) |
| 50% | $161.05 | $20,092.63 | $8,071,805.77 | $(8,051,713.14) |

| National Grid - for 4 events in 2022 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $166.61 | $2,079.31 | $701,009.51 | $(698,930.20) |
| 10% | $166.61 | $4,156.96 | $1,519,986.87 | $(1,515,829.91) |
| 15% | $166.61 | $6,236.28 | $2,338,964.24 | $(2,332,727.96) |
| 25% | $166.61 | $10,393.24 | $3,976,918.96 | $(3,966,525.72) |
| 50% | $166.61 | $20,786.48 | $8,071,805.77 | $(8,051,019.29) |

| National Grid - for 4 events in 2023 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $176.80 | $2,206.53 | $701,009.51 | $(698,802.99) |
| 10% | $176.80 | $4,411.28 | $1,519,986.87 | $(1,515,575.59) |
| 15% | $176.80 | $6,617.81 | $2,338,964.24 | $(2,332,346.43) |
| 25% | $176.80 | $11,029.09 | $3,976,918.96 | $(3,965,889.87) |
| 50% | $176.80 | $22,058.19 | $8,071,805.77 | $(8,049,747.58) |

| National Grid - for 4 events in 2024 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $186.36 | $2,325.71 | $701,009.51 | $(698,683.80) |
| 10% | $186.36 | $4,649.56 | $1,519,986.87 | $(1,515,337.31) |
| 15% | $186.36 | $6,975.27 | $2,338,964.24 | $(2,331,988.96) |
| 25% | $186.36 | $11,624.84 | $3,976,918.96 | $(3,965,294.12) |
| 50% | $186.36 | $23,249.67 | $8,071,805.77 | $(8,048,556.10) |

| National Grid - for 4 events in 2025 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $199.75 | $2,492.83 | $701,009.51 | $(698,516.68) |
| 10% | $199.75 | $4,983.67 | $1,519,986.87 | $(1,515,003.21) |
| 15% | $199.75 | $7,476.50 | $2,338,964.24 | $(2,331,487.73) |
| 25% | $199.75 | $12,460.17 | $3,976,918.96 | $(3,964,458.79) |
| 50% | $199.75 | $24,920.34 | $8,071,805.77 | $(8,046,885.43) |

| National Grid - for 4 events in 2026 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $215.07 | $2,684.02 | $701,009.51 | $(698,325.49) |
| 10% | $215.07 | $5,365.89 | $1,519,986.87 | $(1,514,620.98) |
| 15% | $215.07 | $8,049.92 | $2,338,964.24 | $(2,330,914.32) |
| 25% | $215.07 | $13,415.81 | $3,976,918.96 | $(3,963,503.15) |
| 50% | $215.07 | $26,831.62 | $8,071,805.77 | $(8,044,974.15) |

| National Grid - for 4 events in 2027 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Initial Cost | Economic Benefit |
| 5% | $228.56 | $2,852.44 | $701,009.51 | $(698,157.07) |
| 10% | $228.56 | $5,702.60 | $1,519,986.87 | $(1,514,284.27) |
| 15% | $228.56 | $8,555.05 | $2,338,964.24 | $(2,330,409.19) |
| 25% | $228.56 | $14,257.65 | $3,976,918.96 | $(3,962,661.31) |
| 50% | $228.56 | $28,515.29 | $8,071,805.77 | $(8,043,290.48) |

RoomAC Program: Impact of Energy Efficiency

| NYSEG - for 4 events |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 14.7 | $120.10 | $1,761.67 | $(1,904,138.10) |
| 2019 | 14.7 | $116.06 | $1,702.33 | $(1,904,197.43) |
| 2020 | 14.7 | $101.35 | $1,486.54 | $(1,904,413.23) |
| 2021 | 14.7 | $161.05 | $2,362.29 | $(1,903,537.48) |
| 2022 | 14.7 | $166.61 | $2,443.86 | $(1,903,455.90) |
| 2023 | 14.7 | $176.80 | $2,593.38 | $(1,903,306.39) |
| 2024 | 14.7 | $186.36 | $2,733.46 | $(1,903,166.31) |
| 2025 | 14.7 | $199.75 | $2,929.88 | $(1,902,969.89) |
| 2026 | 14.7 | $215.07 | $3,154.59 | $(1,902,745.18) |
| 2027 | 14.7 | $228.56 | $3,352.54 | $(1,902,547.23) |
| NYSEG - for 4 events |  |  |  |  |
| 10% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 13.9 | $120.10 | $1,668.95 | $(1,904,230.82) |
| 2019 | 13.9 | $116.06 | $1,612.73 | $(1,904,287.03) |
| 2020 | 13.9 | $101.35 | $1,408.30 | $(1,904,491.46) |
| 2021 | 13.9 | $161.05 | $2,237.95 | $(1,903,661.81) |
| 2022 | 13.9 | $166.61 | $2,315.24 | $(1,903,584.53) |
| 2023 | 13.9 | $176.80 | $2,456.88 | $(1,903,442.88) |
| 2024 | 13.9 | $186.36 | $2,589.59 | $(1,903,310.17) |
| 2025 | 13.9 | $199.75 | $2,775.67 | $(1,903,124.09) |
| 2026 | 13.9 | $215.07 | $2,988.56 | $(1,902,911.21) |
| 2027 | 13.9 | $228.56 | $3,176.09 | $(1,902,723.68) |

| NYSEG - for 4 events |  |  |  |  |
| --- | --- | --- | --- | --- |
| 15% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 13.1 | $120.10 | $1,576.23 | $(1,904,323.54) |
| 2019 | 13.1 | $116.06 | $1,523.14 | $(1,904,376.63) |
| 2020 | 13.1 | $101.35 | $1,330.06 | $(1,904,569.70) |
| 2021 | 13.1 | $161.05 | $2,113.62 | $(1,903,786.14) |
| 2022 | 13.1 | $166.61 | $2,186.61 | $(1,903,713.15) |
| 2023 | 13.1 | $176.80 | $2,320.39 | $(1,903,579.37) |
| 2024 | 13.1 | $186.36 | $2,445.73 | $(1,903,454.04) |
| 2025 | 13.1 | $199.75 | $2,621.47 | $(1,903,278.29) |
| 2026 | 13.1 | $215.07 | $2,822.53 | $(1,903,077.24) |
| 2027 | 13.1 | $228.56 | $2,999.64 | $(1,902,900.13) |

| RG&E - for 4 events |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 7.3 | $120.10 | $880.83 | $(56,775.75) |
| 2019 | 7.3 | $116.06 | $851.16 | $(56,805.42) |
| 2020 | 7.3 | $101.35 | $743.27 | $(56,913.32) |
| 2021 | 7.3 | $161.05 | $1,181.14 | $(56,475.44) |
| 2022 | 7.3 | $166.61 | $1,221.93 | $(56,434.66) |
| 2023 | 7.3 | $176.80 | $1,296.69 | $(56,359.90) |
| 2024 | 7.3 | $186.36 | $1,366.73 | $(56,289.86) |
| 2025 | 7.3 | $199.75 | $1,464.94 | $(56,191.65) |
| 2026 | 7.3 | $215.07 | $1,577.29 | $(56,079.29) |
| 2027 | 7.3 | $228.56 | $1,676.27 | $(55,980.32) |
| RG&E - for 4 events |  |  |  |  |
| 10% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 6.9 | $120.10 | $834.47 | $(56,822.11) |
| 2019 | 6.9 | $116.06 | $806.37 | $(56,850.22) |
| 2020 | 6.9 | $101.35 | $704.15 | $(56,952.44) |
| 2021 | 6.9 | $161.05 | $1,118.98 | $(56,537.61) |
| 2022 | 6.9 | $166.61 | $1,157.62 | $(56,498.97) |
| 2023 | 6.9 | $176.80 | $1,228.44 | $(56,428.15) |
| 2024 | 6.9 | $186.36 | $1,294.80 | $(56,361.79) |
| 2025 | 6.9 | $199.75 | $1,387.84 | $(56,268.75) |
| 2026 | 6.9 | $215.07 | $1,494.28 | $(56,162.31) |
| 2027 | 6.9 | $228.56 | $1,588.04 | $(56,068.54) |

| RG&E - for 4 events |  |  |  |  |
| --- | --- | --- | --- | --- |
| 15% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 6.6 | $120.10 | $788.11 | $(56,868.47) |
| 2019 | 6.6 | $116.06 | $761.57 | $(56,895.02) |
| 2020 | 6.6 | $101.35 | $665.03 | $(56,991.56) |
| 2021 | 6.6 | $161.05 | $1,056.81 | $(56,599.78) |
| 2022 | 6.6 | $166.61 | $1,093.31 | $(56,563.28) |
| 2023 | 6.6 | $176.80 | $1,160.19 | $(56,496.39) |
| 2024 | 6.6 | $186.36 | $1,222.86 | $(56,433.72) |
| 2025 | 6.6 | $199.75 | $1,310.73 | $(56,345.85) |
| 2026 | 6.6 | $215.07 | $1,411.26 | $(56,245.32) |
| 2027 | 6.6 | $228.56 | $1,499.82 | $(56,156.77) |

| National Grid - for 4 events |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 11.9 | $120.10 | $1,423.94 | $(699,585.58) |
| 2019 | 11.9 | $116.06 | $1,375.98 | $(699,633.54) |
| 2020 | 11.9 | $101.35 | $1,201.55 | $(699,807.96) |
| 2021 | 11.9 | $161.05 | $1,909.41 | $(699,100.10) |
| 2022 | 11.9 | $166.61 | $1,975.35 | $(699,034.16) |
| 2023 | 11.9 | $176.80 | $2,096.20 | $(698,913.31) |
| 2024 | 11.9 | $186.36 | $2,209.43 | $(698,800.08) |
| 2025 | 11.9 | $199.75 | $2,368.19 | $(698,641.32) |
| 2026 | 11.9 | $215.07 | $2,549.82 | $(698,459.69) |
| 2027 | 11.9 | $228.56 | $2,709.82 | $(698,299.69) |
| National Grid - for 4 events |  |  |  |  |
| 10% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 11.2 | $120.10 | $1,348.99 | $(699,660.52) |
| 2019 | 11.2 | $116.06 | $1,303.56 | $(699,705.96) |
| 2020 | 11.2 | $101.35 | $1,138.31 | $(699,871.20) |
| 2021 | 11.2 | $161.05 | $1,808.92 | $(699,200.60) |
| 2022 | 11.2 | $166.61 | $1,871.38 | $(699,138.13) |
| 2023 | 11.2 | $176.80 | $1,985.87 | $(699,023.64) |
| 2024 | 11.2 | $186.36 | $2,093.14 | $(698,916.37) |
| 2025 | 11.2 | $199.75 | $2,243.55 | $(698,765.96) |
| 2026 | 11.2 | $215.07 | $2,415.62 | $(698,593.89) |
| 2027 | 11.2 | $228.56 | $2,567.20 | $(698,442.31) |

| National Grid - for 4 events |  |  |  |  |
| --- | --- | --- | --- | --- |
| 15% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 10.6 | $120.10 | $1,274.05 | $(699,735.46) |
| 2019 | 10.6 | $116.06 | $1,231.14 | $(699,778.38) |
| 2020 | 10.6 | $101.35 | $1,075.07 | $(699,934.44) |
| 2021 | 10.6 | $161.05 | $1,708.42 | $(699,301.09) |
| 2022 | 10.6 | $166.61 | $1,767.42 | $(699,242.09) |
| 2023 | 10.6 | $176.80 | $1,875.55 | $(699,133.96) |
| 2024 | 10.6 | $186.36 | $1,976.86 | $(699,032.66) |
| 2025 | 10.6 | $199.75 | $2,118.91 | $(698,890.60) |
| 2026 | 10.6 | $215.07 | $2,281.42 | $(698,728.09) |
| 2027 | 10.6 | $228.56 | $2,424.58 | $(698,584.93) |

RoomAC Program: Summary

Due to a relatively low average reduction of 0.0016 kW per event per participant between NYSEG and RG&E, this program was not economically viable for any year. Even if the initial cost was $1, for 5% enrollment in 2017, the program is still not economically viable. The economic benefit could only be increased by increasing the number of events per year. It is not recommended for National Grid to implement this program.

For NYSEG, this program requires at least 3,581events to become economically viable, for 5% enrollment in 2017. For RG&E, this program requires at least 1,950 events to become economically viable, for 5% enrollment in 2017. If National Grid were to implement this program, it would require at least 404 events to be economically viable, for 5% enrollment in 2017.

CSRP Program

| NYSEG |  |  |
| --- | --- | --- |
| Enrollment | Commercial Count | Estimated kW Relief per Event |
| 1% | 1,160 | 481,011 |
| 2% | 2,320 | 962,022 |
| 5% | 5,800 | 2,405,056 |
| 10% | 11,600 | 4,810,112 |
| 15% | 17,401 | 7,215,582 |

| RG&E |  |  |
| --- | --- | --- |
| Enrollment | Commercial Count | Estimated kW Relief per Event |
| 1% | 419 | 57,708 |
| 2% | 838 | 116,790 |
| 5% | 2,096 | 290,870 |
| 10% | 4,192 | 581,741 |
| 15% | 6,288 | 872,611 |

| Event Description | - Events have taken place from 0 to 15 times per year, since 2015. |
| --- | --- |
| Requirements | - Commercial customers can either apply through an aggregator, or through the utility as a direct participant if they are able to reduce their electricity use by 50 kW or more. |
| Incentives | - For the Voluntary Participation Option, the performance amount paid per event is equal to the applicable performance payment rate per kilowatt-hour (kWh) multiplied by the average kilowatt (kW) of load relief provided during the event multiplied by the number of event hours. The current performance payment rate is $0.15 per kWh. For the Reservation Payment Option, the monthly payment is calculated by multiplying the applicable reservation payment rate per kW by the kW of contracted load relief multiplied by performance factor for the month. There is a reservation payment rate for months where we have requested four or fewer planned load relief events. There is also a reservation payment rate for months where we have requested five or more planned load relief events. The current reservation payment rate for four or less planned events is $3.25 per kW per month. The current reservation payment rate for five or more planned events is $3.50 per kW per month. |

Cost

Potential (per participant per event)

$0.15/kWh, $3.25/kWh, or $3.50/kWh

| NYSEG - for 1 event in 2017 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $137.88 | $66,323,133 | $72,152 | $66,250,981 |
| 2% | $137.88 | $132,646,265 | $144,303 | $132,501,962 |
| 5% | $137.88 | $331,615,801 | $360,758 | $331,255,042 |
| 10% | $137.88 | $663,231,601 | $721,517 | $662,510,085 |
| 15% | $137.88 | $994,904,486 | $1,082,337 | $993,822,148 |

| NYSEG - for 1 event in 2018 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $120.10 | $57,774,020 | $72,156 | $57,701,864 |
| 2% | $120.10 | $115,548,040 | $144,312 | $115,403,728 |
| 5% | $120.10 | $288,853,526 | $360,758 | $288,492,768 |
| 10% | $120.10 | $577,707,052 | $721,517 | $576,985,535 |
| 15% | $120.10 | $866,610,301 | $1,082,337 | $865,527,963 |

| NYSEG - for 1 event in 2019 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $116.06 | $55,824,904 | $72,152 | $55,752,752 |
| 2% | $116.06 | $111,649,807 | $144,303 | $111,505,504 |
| 5% | $116.06 | $279,124,518 | $360,758 | $278,763,760 |
| 10% | $116.06 | $558,249,037 | $721,517 | $557,527,520 |
| 15% | $116.06 | $837,421,603 | $1,082,337 | $836,339,266 |

| NYSEG - for 1 event in 2020 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $101.35 | $48,748,388 | $72,152 | $48,676,236 |
| 2% | $101.35 | $97,496,776 | $144,303 | $97,352,473 |
| 5% | $101.35 | $243,741,940 | $360,758 | $243,381,182 |
| 10% | $101.35 | $487,483,881 | $721,517 | $486,762,364 |
| 15% | $101.35 | $731,267,778 | $1,082,337 | $730,185,441 |

| NYSEG - for 1 event in 2021 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $161.05 | $77,466,988 | $72,152 | $77,394,837 |
| 2% | $161.05 | $154,933,976 | $144,303 | $154,789,673 |
| 5% | $161.05 | $387,334,941 | $360,758 | $386,974,183 |
| 10% | $161.05 | $774,669,882 | $721,517 | $773,948,366 |
| 15% | $161.05 | $1,162,071,498 | $1,082,337 | $1,160,989,161 |

| NYSEG - for 4 events in 2022 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $166.61 | $80,142,114 | $72,152 | $80,069,963 |
| 2% | $166.61 | $160,284,229 | $144,303 | $160,139,925 |
| 5% | $166.61 | $400,710,572 | $360,758 | $400,349,813 |
| 10% | $166.61 | $801,421,143 | $721,517 | $800,699,627 |
| 15% | $166.61 | $1,202,200,692 | $1,082,337 | $1,201,118,355 |

| NYSEG - for 1 event in 2023 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $176.80 | $85,045,168 | $72,152 | $84,973,016 |
| 2% | $176.80 | $170,090,335 | $144,303 | $169,946,032 |
| 5% | $176.80 | $425,225,838 | $360,758 | $424,865,080 |
| 10% | $176.80 | $850,451,677 | $721,517 | $849,730,160 |
| 15% | $176.80 | $1,275,750,712 | $1,082,337 | $1,274,668,375 |

| NYSEG - for 1 event in 2024 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $186.36 | $89,638,930 | $72,152 | $89,566,778 |
| 2% | $186.36 | $179,277,860 | $144,303 | $179,133,557 |
| 5% | $186.36 | $448,194,651 | $360,758 | $447,833,892 |
| 10% | $186.36 | $896,389,302 | $721,517 | $895,667,785 |
| 15% | $186.36 | $1,344,661,103 | $1,082,337 | $1,343,578,766 |

| NYSEG - for 1 event in 2025 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $199.75 | $96,080,173 | $72,152 | $96,008,021 |
| 2% | $199.75 | $192,160,346 | $144,303 | $192,016,043 |
| 5% | $199.75 | $480,400,865 | $360,758 | $480,040,107 |
| 10% | $199.75 | $960,801,731 | $721,517 | $960,080,214 |
| 15% | $199.75 | $1,441,285,291 | $1,082,337 | $1,440,202,954 |

| NYSEG - for 1 event in 2026 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $215.07 | $103,449,118 | $72,152 | $103,376,967 |
| 2% | $215.07 | $206,898,237 | $144,303 | $206,753,934 |
| 5% | $215.07 | $517,245,592 | $360,758 | $516,884,834 |
| 10% | $215.07 | $1,034,491,185 | $721,517 | $1,033,769,668 |
| 15% | $215.07 | $1,551,825,815 | $1,082,337 | $1,550,743,477 |

| NYSEG - for 1 event in 2027 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $228.56 | $109,940,489 | $72,152 | $109,868,337 |
| 2% | $228.56 | $219,880,978 | $144,303 | $219,736,675 |
| 5% | $228.56 | $549,702,445 | $360,758 | $549,341,686 |
| 10% | $228.56 | $1,099,404,890 | $721,517 | $1,098,683,373 |
| 15% | $228.56 | $1,649,201,959 | $1,082,337 | $1,648,119,622 |

| RG&E - for 1 event in 2017 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $137.88 | $7,956,939.31 | $8,656.20 | $7,948,283.11 |
| 2% | $137.88 | $16,103,329.55 | $17,518.50 | $16,085,811.05 |
| 5% | $137.88 | $40,105,963.42 | $43,630.50 | $40,062,332.92 |
| 10% | $137.88 | $80,212,064.72 | $87,261.15 | $80,124,803.57 |
| 15% | $137.88 | $120,318,028.13 | $130,891.65 | $120,187,136.48 |

| RG&E - for 1 event in 2018 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $120.10 | $6,986,849.80 | $8,726.10 | $6,978,123.70 |
| 2% | $120.10 | $13,973,699.59 | $17,452.20 | $13,956,247.39 |
| 5% | $120.10 | $34,934,248.98 | $43,630.50 | $34,890,618.48 |
| 10% | $120.10 | $69,868,618.07 | $87,261.15 | $69,781,356.92 |
| 15% | $120.10 | $104,802,867.05 | $130,891.65 | $104,671,975.40 |

| RG&E - for 1 event in 2019 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $116.06 | $6,751,522.52 | $8,726.10 | $6,742,796.42 |
| 2% | $116.06 | $13,503,045.03 | $17,452.20 | $13,485,592.83 |
| 5% | $116.06 | $33,757,612.58 | $43,630.50 | $33,713,982.08 |
| 10% | $116.06 | $67,515,341.23 | $87,261.15 | $67,428,080.08 |
| 15% | $116.06 | $101,272,953.81 | $130,891.65 | $101,142,062.16 |

| RG&E - for 1 event in 2020 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $101.35 | $5,895,681.28 | $8,726.10 | $5,886,955.18 |
| 2% | $101.35 | $11,791,362.57 | $17,452.20 | $11,773,910.37 |
| 5% | $101.35 | $29,478,406.41 | $43,630.50 | $29,434,775.91 |
| 10% | $101.35 | $58,956,914.18 | $87,261.15 | $58,869,653.03 |
| 15% | $101.35 | $88,435,320.59 | $130,891.65 | $88,304,428.94 |

| RG&E - for 1 event in 2021 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $161.05 | $9,368,938.96 | $8,726.10 | $9,360,212.86 |
| 2% | $161.05 | $18,737,877.93 | $17,452.20 | $18,720,425.73 |
| 5% | $161.05 | $46,844,694.82 | $43,630.50 | $46,801,064.32 |
| 10% | $161.05 | $93,689,550.68 | $87,261.15 | $93,602,289.53 |
| 15% | $161.05 | $140,534,245.50 | $130,891.65 | $140,403,353.85 |

| RG&E - for 1 event in 2022 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $166.61 | $9,692,472 | $8,726 | $9,683,745 |
| 2% | $166.61 | $19,384,943 | $17,452 | $19,367,491 |
| 5% | $166.61 | $48,462,358 | $43,631 | $48,418,727 |
| 10% | $166.61 | $96,924,882 | $87,261 | $96,837,621 |
| 15% | $166.61 | $145,387,239 | $130,892 | $145,256,348 |

| RG&E - for 1 event in 2023 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $176.80 | $10,285,452 | $8,726 | $10,276,726 |
| 2% | $176.80 | $20,570,904 | $17,452 | $20,553,452 |
| 5% | $176.80 | $51,427,260 | $43,631 | $51,383,629 |
| 10% | $176.80 | $102,854,696 | $87,261 | $102,767,435 |
| 15% | $176.80 | $154,281,956 | $130,892 | $154,151,064 |

| RG&E - for 1 event in 2024 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $186.36 | $10,841,026 | $8,726 | $10,832,300 |
| 2% | $186.36 | $21,682,053 | $17,452 | $21,664,601 |
| 5% | $186.36 | $54,205,132 | $43,631 | $54,161,502 |
| 10% | $186.36 | $108,410,450 | $87,261 | $108,323,189 |
| 15% | $186.36 | $162,615,583 | $130,892 | $162,484,691 |

| RG&E - for 1 event in 2025 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $199.75 | $11,620,037 | $8,726 | $11,611,311 |
| 2% | $199.75 | $23,240,074 | $17,452 | $23,222,622 |
| 5% | $199.75 | $58,100,185 | $43,631 | $58,056,555 |
| 10% | $199.75 | $116,200,571 | $87,261 | $116,113,310 |
| 15% | $199.75 | $174,300,756 | $130,892 | $174,169,865 |

| RG&E - for 1 event in 2026 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $215.07 | $12,511,245 | $8,726 | $12,502,519 |
| 2% | $215.07 | $25,022,490 | $17,452 | $25,005,038 |
| 5% | $215.07 | $62,556,226 | $43,631 | $62,512,595 |
| 10% | $215.07 | $125,112,666 | $87,261 | $125,025,405 |
| 15% | $215.07 | $187,668,892 | $130,892 | $187,538,000 |

| RG&E - for 1 event in 2027 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Enrollment | AGCC | Event Savings | Event Cost | Economic Benefit |
| 1% | $228.56 | $13,296,318 | $8,726 | $13,287,592 |
| 2% | $228.56 | $26,592,637 | $17,452 | $26,575,184 |
| 5% | $228.56 | $66,481,591 | $43,631 | $66,437,961 |
| 10% | $228.56 | $132,963,411 | $87,261 | $132,876,150 |
| 15% | $228.56 | $199,445,003 | $130,892 | $199,314,111 |

CSRP Program: Impact of Energy Efficiency

| NYSEG - for 1 event |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 456,960 | $120.10 | $54,882,147.13 | $54,809,995.48 |
| 2019 | 456,960 | $116.06 | $53,033,636.46 | $52,961,484.81 |
| 2020 | 456,960 | $101.35 | $46,310,949.43 | $46,238,797.78 |
| 2021 | 456,960 | $161.05 | $73,593,608.22 | $73,521,456.57 |
| 2022 | 456,960 | $166.61 | $76,134,976.96 | $76,062,825.31 |
| 2023 | 456,960 | $176.80 | $80,792,875.69 | $80,720,724.04 |
| 2024 | 456,960 | $186.36 | $85,156,948.24 | $85,084,796.59 |
| 2025 | 456,960 | $199.75 | $91,276,126.47 | $91,203,974.82 |
| 2026 | 456,960 | $215.07 | $98,276,621.71 | $98,204,470.06 |
| 2027 | 456,960 | $228.56 | $104,443,421.09 | $104,371,269.44 |
| NYSEG - for 1 event |  |  |  |  |
| 10% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 432,910 | $120.10 | $51,993,613.07 | $51,921,461.42 |
| 2019 | 432,910 | $116.06 | $50,242,392.44 | $50,170,240.79 |
| 2020 | 432,910 | $101.35 | $43,873,531.04 | $43,801,379.39 |
| 2021 | 432,910 | $161.05 | $69,720,260.42 | $69,648,108.77 |
| 2022 | 432,910 | $166.61 | $72,127,872.91 | $72,055,721.26 |
| 2023 | 432,910 | $176.80 | $76,540,619.07 | $76,468,467.42 |
| 2024 | 432,910 | $186.36 | $80,675,003.60 | $80,602,851.95 |
| 2025 | 432,910 | $199.75 | $86,472,119.81 | $86,399,968.16 |
| 2026 | 432,910 | $215.07 | $93,104,167.94 | $93,032,016.29 |
| 2027 | 432,910 | $228.56 | $98,946,398.93 | $98,874,247.28 |
| NYSEG - for 1 event |  |  |  |  |
| 15% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 408,859 | $120.10 | $49,105,079.01 | $49,032,927.36 |
| 2019 | 408,859 | $116.06 | $47,451,148.41 | $47,378,996.76 |
| 2020 | 408,859 | $101.35 | $41,436,112.65 | $41,363,961.00 |
| 2021 | 408,859 | $161.05 | $65,846,912.62 | $65,774,760.97 |
| 2022 | 408,859 | $166.61 | $68,120,768.86 | $68,048,617.21 |
| 2023 | 408,859 | $176.80 | $72,288,362.46 | $72,216,210.81 |
| 2024 | 408,859 | $186.36 | $76,193,058.95 | $76,120,907.30 |
| 2025 | 408,859 | $199.75 | $81,668,113.15 | $81,595,961.50 |
| 2026 | 408,859 | $215.07 | $87,931,714.16 | $87,859,562.51 |
| 2027 | 408,859 | $228.56 | $93,449,376.77 | $93,377,225.12 |
| RG&E - for 1 event |  |  |  |  |
| 5% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 54,823 | $120.10 | $6,584,337.88 | $6,575,681.68 |
| 2019 | 54,823 | $116.06 | $6,362,567.79 | $6,353,911.59 |
| 2020 | 54,823 | $101.35 | $5,556,031.50 | $5,547,375.30 |
| 2021 | 54,823 | $161.05 | $8,829,195.06 | $8,820,538.86 |
| 2022 | 54,823 | $166.61 | $9,134,088.93 | $9,125,432.73 |
| 2023 | 54,823 | $176.80 | $9,692,907.79 | $9,684,251.59 |
| 2024 | 54,823 | $186.36 | $10,216,475.65 | $10,207,819.45 |
| 2025 | 54,823 | $199.75 | $10,950,607.59 | $10,941,951.39 |
| 2026 | 54,823 | $215.07 | $11,790,473.16 | $11,781,816.96 |
| 2027 | 54,823 | $228.56 | $12,530,318.32 | $12,521,662.12 |
| RG&E - for 1 event |  |  |  |  |
| 10% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 51,937 | $120.10 | $6,237,793.78 | $6,229,137.58 |
| 2019 | 51,937 | $116.06 | $6,027,695.80 | $6,019,039.60 |
| 2020 | 51,937 | $101.35 | $5,263,608.79 | $5,254,952.59 |
| 2021 | 51,937 | $161.05 | $8,364,500.58 | $8,355,844.38 |
| 2022 | 51,937 | $166.61 | $8,653,347.41 | $8,644,691.21 |
| 2023 | 51,937 | $176.80 | $9,182,754.75 | $9,174,098.55 |
| 2024 | 51,937 | $186.36 | $9,678,766.41 | $9,670,110.21 |
| 2025 | 51,937 | $199.75 | $10,374,259.82 | $10,365,603.62 |
| 2026 | 51,937 | $215.07 | $11,169,921.94 | $11,161,265.74 |
| 2027 | 51,937 | $228.56 | $11,870,827.88 | $11,862,171.68 |
| RG&E - for 1 event |  |  |  |  |
| 15% reduction in demand (due to a potential increase in energy efficiency) |  |  |  |  |
| Year | kW Relief | AGCC | Event Savings | Economic Benefit |
| 2018 | 49,052 | $120.10 | $5,891,249.68 | $5,882,593.48 |
| 2019 | 49,052 | $116.06 | $5,692,823.81 | $5,684,167.61 |
| 2020 | 49,052 | $101.35 | $4,971,186.08 | $4,962,529.88 |
| 2021 | 49,052 | $161.05 | $7,899,806.10 | $7,891,149.90 |
| 2022 | 49,052 | $166.61 | $8,172,605.89 | $8,163,949.69 |
| 2023 | 49,052 | $176.80 | $8,672,601.71 | $8,663,945.51 |
| 2024 | 49,052 | $186.36 | $9,141,057.16 | $9,132,400.96 |
| 2025 | 49,052 | $199.75 | $9,797,912.05 | $9,789,255.85 |
| 2026 | 49,052 | $215.07 | $10,549,370.72 | $10,540,714.52 |
| 2027 | 49,052 | $228.56 | $11,211,337.44 | $11,202,681.24 |

CSRP Program: Summary

For both NYSEG and RG&E, this program is economically viable for all years. For all levels of increased energy efficiency (5, 10, and 15 percent), this program remains economically viable for all years, for both utilities.

National Grid

Emergency/Day Ahead Demand Programs

| National Grid |  |  |
| --- | --- | --- |
| Enrollment | Commercial Count | Estimated kW Relief per Event |
| 1% | 572 | 295,735 |
| 2% | 1,143 | 590,953 |
| 5% | 2,856 | 1,476,607 |
| 10% | 5,712 | 2,953,214 |
| 15% | 8,568 | 4,429,821 |

| Event Description | - It is unknown how many events typically take place in one year, so one event will be considered. Emergency Demand: as participants will receive a financial incentive if they can curtail at least 100 kW of electricity one hour after notification, it will be assumed that one event is one hour in duration. |
| --- | --- |
| Requirements | Commercial participants must be able to curtail at least 100 kW, as well as meet rate class, interval metering and on-site generation requirements. |
| Incentives | - Emergency Demand: The NYISO will pay National Grid the greater of $500 MWH or the real time Location Based Marginal Price (LBMP). National Grid will pay 90 percent of the NYISO payment for your company's actual kWh reduction. Day Ahead Demand: The NYISO will pay National Grid the day ahead LBMP times the customers’ bid reduction. National Grid will pay participating customers 90 percent of the NYISO payment for their bid reduction. Payments will only be made for load reductions that are not provided from the use of on-site generation. |

As the LBMP fluctuates, it will be assumed that all customer participants are only participating in the Emergency Demand program. It will be assumed that $500 per MWh is the more expensive price that NYISO will pay.

Economic Gain

Potential (per event)

$50/MWh

| National Grid - for 1 event |  |  |
| --- | --- | --- |
| Enrollment | MW Reduction | Economic Benefit |
| 1% | 296 | $14,786.75 |
| 2% | 591 | $29,547.65 |
| 5% | 1,477 | $73,830.36 |
| 10% | 2,953 | $147,660.72 |
| 15% | 4,430 | $221,491.07 |

CSRP Program: Impact of Energy Efficiency

| National Grid - for 1 event |  |  |
| --- | --- | --- |
| Demand Reduction | MW Reduction | Economic Benefit |
| 5% | 281 | $14,047.41 |
| 10% | 266 | $13,308.08 |
| 15% | 251 | $12,568.74 |

Day Ahead Demand Program: Summary

For National Grid, this program is economically viable for all years. For all levels of increased energy efficiency (5, 10, and 15 percent), this program remains economically viable for all years.

CONCLUSION

Commercial programs are economically viable for every year, for each utility, even with potential improvements in energy efficiency. Residential programs had varying results across the three utilities, and should only be implemented going forward with the consideration of recommended improvements (increasing the number of events per year, targeting residents within higher zones of AGCC, or limiting the number of participants), to ensure economic viability for every year. With the adaptation of Time of Use (TOU) pricing, where customers engage with dynamic pricing that appropriately responds to periods of high demand, it is unlikely that DR programs will be necessary in the future. Until smart meters and TOU pricing are fully implemented in New York, DR programs, as they are currently structured and implemented, are generally an economically viable way to shed load and increase grid reliability.

Smart Savings Rewards, https://enrollmythermostat.com

Room Air Conditioner Control Program, http://www.nyseg.com/YourBusiness/demandresponse/accontrol.html

Commercial System Relief Program, https://www.rge.com/YourBusiness/demandresponse/commercialsystem.html

Distribution Load Relief Program, https://www.rge.com/YourBusiness/demandresponse/distributionloadrelief.html

Emergency Demand, https://www9.nationalgridus.com/niagaramohawk/business/programs/4_emergency.asp

Day Ahead Demand, https://www9.nationalgridus.com/niagaramohawk/business/programs/4_dayahead.asp

Total “Number of Devices/Participants Participating in Event(s)” assumes that one participant is responsible for each device for BYOT/DIT and RoomAC programs.

Participants are either direct customers or aggregators.

CSRP had two years of data, so the estimated kW relief per device per event was estimated for each year based on the number of devices and the average kW relief per event of each year. These values were then averaged to find the estimated kW relief per device per event for the program overall.

Participants are either direct customers or aggregators.

CSRP had two years of data, so the estimated kW relief per device per event was estimated for each year based on the number of devices and the average kW relief per event of each year. These values were then averaged to find the estimated kW relief per device per event for the program overall.

CSRP had two years of data, so the estimated kW relief per device per event was estimated for each year based on the number of devices and the average kW relief per event of each year. These values were then averaged to find the estimated kW relief per device per event for the program overall.

Data was provided by NYSEG and RG&E in a document titled “Service Address Zip Codes for NYSEG and RGE Accounts with count.” Account IDs with ambiguous labels, like “Intercompany Borderline Sale for Resale Tax exempt,” are not included in the customer count. National Grid data was estimated from “Building Stock” data provided in the “NY MME- Renewables template,” and that the average office building is 14,900 square feet. Source: http://www.apep.uci.edu/der/buildingintegration/2/BuildingTemplates/Office.aspx

These values were obtained via extrapolation with the average of the number of devices and the estimated kW relief per device obtained in the analysis of NYSEG and RG&E’s CSRP. The results were multiplied by 2, as National Grid requires a 100 kW reduction, instead of a 50 kW reduction, and are based on National Grid’s estimated customer count.

Feldman, B. (2017, March 31). Natural Gas Demand Response – Not Just for Electricity Any More: Part 1. https://www.navigantresearch.com/blog/natural-gas-demand-response-not-just-for-electricity-any-more-part-one

It will be assumed that all reduction during a Demand Response event will be peak load reduction.

This data was provided by RG&E and NYSEG. The AGCC was averaged for all four zones: NYC, Lower Hudson Valley, Long Island, and Rest of State. All costs are in 2017 dollars.

The initial cost of customers already enrolled was subtracted from the calculations. NYSEG has 2,073 customers enrolled in the BYOT program, and 120 customers enrolled in the DIT program.

Economic benefit for each year includes the initial cost for each new participant as if they had not previously enrolled. In other words, the initial cost is not assumed to be already paid in years beyond 2017.

The initial cost of customers already enrolled was subtracted from the calculations. RG&E has 2,297 customers enrolled in the BYOT program, and 54 customers enrolled in the DIT program.

Economic benefit for each year includes the initial cost for each new participant as if they had not previously enrolled. In other words, the initial cost is not assumed to be already paid in years beyond 2017.

Economic benefit for each year includes the initial cost for each new participant as if they had not previously enrolled. In other words, the initial cost is not assumed to be already paid in years beyond 2017.

Only 5% enrollment will be considered in the estimate of the impact of energy efficiency on this program.

Economic benefit for each year includes the initial cost for each new participant as if they had not previously enrolled. In other words, the initial cost is not assumed to be already paid in years beyond 2017.

Economic benefit for each year includes the initial cost for each new participant as if they had not previously enrolled. In other words, the initial cost is not assumed to be already paid in years beyond 2017.

Economic benefit for each year includes the initial cost for each new participant as if they had not previously enrolled. In other words, the initial cost is not assumed to be already paid in years beyond 2017.

The cost is based on the thinkeco WiFi smartAC kit, which is priced at $139.99. Source: https://shop.thinkecoinc.com/products/wifi-smartac-kit#.Wf-c_xNSzVo

The initial cost of customers already enrolled was subtracted from the calculations. NYSEG has 1,315 customers enrolled in the RoomAC program.

Economic benefit for each year includes the initial cost for each new participant as if they had not previously enrolled. In other words, the initial cost is not assumed to be already paid in years beyond 2017.

The initial cost of customers already enrolled was subtracted from the calculations. RG&E has 715 customers enrolled in the RoomAC program.

Economic benefit for each year includes the initial cost for each new participant as if they had not previously enrolled. In other words, the initial cost is not assumed to be already paid in years beyond 2017.

Economic benefit for each year includes the initial cost for each new participant as if they had not previously enrolled. In other words, the initial cost is not assumed to be already paid in years beyond 2017.

Only 5% enrollment will be considered in the estimate of the impact of energy efficiency on this program.

Economic benefit for each year includes the initial cost for each new participant as if they had not previously enrolled. In other words, the initial cost is not assumed to be already paid in years beyond 2017.

Economic benefit for each year includes the initial cost for each new participant as if they had not previously enrolled. In other words, the initial cost is not assumed to be already paid in years beyond 2017.

Economic benefit for each year includes the initial cost for each new participant as if they had not previously enrolled. In other words, the initial cost is not assumed to be already paid in years beyond 2017.

All participants were assumed to elect the Voluntary Participation Option.

All participants were assumed to elect the Voluntary Participation Option.

Only 1% enrollment will be considered in the estimate of the impact of energy efficiency on this program.

Only 1% enrollment will be considered in the estimate of the impact of energy efficiency on this program.